EMBEDDED SYSTEMS & IoT

Complete Student Workshop Notes

Arduino  ·  ESP8266 / ESP32  ·  Raspberry Pi

| About This Guide | Details |
| --- | --- |
| Total Sessions | 8 comprehensive sessions |
| Duration | 16 – 20 hours of learning |
| Level | Beginner — no prior electronics needed |
| Platforms | Arduino Uno/Nano, ESP8266, ESP32, Raspberry Pi 4 |
| Goal | Build your first real IoT project independently |
| What You Will Learn | Hardware, software, firmware, protocols, sensors, wireless, and more |

HOW TO USE THESE NOTES

- 🔑  Yellow boxes — Key concepts to memorise

- 🛠️  Teal boxes — Hands-on exercises to try yourself

- ⚠️  Red boxes — Warnings — mistakes that burn hardware or crash code

- 💡  Green boxes — Pro tips to level up your skills

- Code blocks — Copy these directly into your IDE

# Table of Contents

Session 01  Hardware Fundamentals	3

Session 02  Pins — Digital, Analogue & PWM	10

Session 03  Boot, Reset & Memory	16

Session 04  Code, Firmware & the IDE	21

Session 05  Serial, Monitor, Plotter & Debugging	29

Session 06  Communication Protocols	35

Session 07  Interfaces, Displays & Sensors	44

Session 08  Platform-Specific & Going Further	51

Appendix A  Quick Reference Cheat Sheet	59

Appendix B  Glossary of Terms	62

| SESSION 01 | Hardware Fundamentals Understanding the building blocks of every embedded system |
| --- | --- |

By the end of this session you will be able to: identify microcontroller and microprocessor boards, explain how voltage logic levels work, understand power pins and current limits, read a resistor colour code, wire a breadboard, and explain why pull resistors are necessary.

## 1.1  Microcontroller vs Microprocessor

Before touching any hardware you need to understand the fundamental difference between these two types of chips. Confusing them is the single most common mistake beginners make.

Microprocessor (MPU)

A microprocessor is only a Central Processing Unit (CPU). It is the brain, but it has no memory and no input/output circuitry built in. To be useful it needs external chips for RAM (to store running data), Flash or hard disk (to store programs), and peripheral controllers (to talk to buttons, screens, etc.). Your laptop contains a microprocessor. The Raspberry Pi 4 uses a quad-core ARM Cortex-A72 microprocessor chip.

Microcontroller (MCU)

A microcontroller packs the CPU, RAM, Flash storage, and peripheral controllers (GPIO, UART, SPI, I2C, ADC, timers) all onto a single chip. This makes it cheap, small, and extremely power-efficient. When you buy an Arduino Uno, the heart of it is the ATmega328P — a microcontroller.

🔑  Key Concept:  Microcontroller = CPU + Memory + Peripherals on one chip. Microprocessor = CPU only.

| Property | Microcontroller (MCU) vs Microprocessor (MPU) |
| --- | --- |
| Example | ATmega328P, ESP32, STM32 / Intel Core, ARM Cortex-A |
| RAM | 2 KB – 512 KB / 1 GB – 32 GB |
| Storage | 32 KB – 16 MB Flash on chip / SSD, SD card (GBs) |
| OS | None (bare metal) or tiny RTOS / Full Linux, Windows |
| Boot time | Under 1 second / 10 – 60 seconds |
| Power draw | 1 mA – 500 mA / 2 W – 100 W |
| Cost | $1 – $15 / $20 – $200+ |
| Board examples | Arduino Uno, ESP32 DevKit, NodeMCU / Raspberry Pi 4, Jetson Nano |

## 1.2  The Three Platforms You Will Use

### Arduino Uno and Nano

The Arduino Uno is the ideal starting point for anyone learning embedded systems. It is based on the 8-bit ATmega328P microcontroller running at 16 MHz. The Nano is a smaller version of the Uno with the same chip, perfect for fitting into tight projects.

- Processor: ATmega328P, 8-bit AVR, 16 MHz

- Memory: 32 KB Flash (program), 2 KB SRAM (runtime), 1 KB EEPROM (persistent storage)

- Logic voltage: 5 V — this is higher than most modern boards

- GPIO pins: 14 digital I/O (6 with PWM), 6 analogue input

- USB chip: ATmega16U2 on Uno (genuine), CH340/CP2102 on cheap clones — may need a driver

- Best for: Learning fundamentals, 5 V sensor world, motor/relay control

💡  Pro Tip:  Arduino is beginner-friendly because it is tolerant of mistakes and has the largest online community. If something breaks you will almost always find the answer on the Arduino Forum.

### ESP8266 — NodeMCU and Wemos D1 Mini

The ESP8266 is a WiFi-capable microcontroller made by Espressif Systems. It is far more powerful than the Arduino Uno and has built-in WiFi, making it the backbone of millions of IoT devices worldwide.

- Processor: Xtensa LX106, 32-bit, 80 MHz (can run at 160 MHz)

- Memory: ~80 KB usable SRAM, 4 MB Flash (on most modules)

- Logic voltage: 3.3 V — this is critical, see warning below

- Wireless: Built-in 2.4 GHz WiFi 802.11 b/g/n

- Analogue input: Only ONE analogue pin (A0), and it accepts a maximum of 1.0 V on the chip (the NodeMCU breakout has a voltage divider to allow up to 3.3 V)

⚠️  Warning:  The ESP8266 operates at 3.3 V logic. Connecting a 5 V Arduino signal directly to any ESP8266 GPIO pin WILL destroy it permanently. Always use a logic level shifter or voltage divider when mixing 5 V and 3.3 V devices.

### ESP32

The ESP32 is the big brother of the ESP8266. It is a dual-core, 32-bit microcontroller with both WiFi and Bluetooth built in. It is the go-to choice for serious IoT projects.

- Processor: Dual-core Xtensa LX6, up to 240 MHz per core

- Memory: 520 KB SRAM, 4 MB Flash, optional external PSRAM (up to 8 MB)

- Logic voltage: 3.3 V

- Wireless: WiFi 802.11 b/g/n + Bluetooth Classic 4.2 + BLE 4.2

- Special features: Hardware PWM on all output pins, 2× true DAC outputs, capacitive touch sensors, Hall effect sensor, ULP co-processor for ultra-low-power operation

- ADC: Two ADC units, 12-bit resolution (0–4095), but WiFi sharing with ADC2 causes issues — prefer ADC1 pins (GPIO32–39)

### Raspberry Pi 4 Model B

The Raspberry Pi is not a microcontroller — it is a full single-board computer running Linux. Understanding this distinction is essential. It has a real operating system, a file system, multiple cores, gigabytes of RAM, and can run Python, Node.js, or any Linux application.

- Processor: Quad-core ARM Cortex-A72, 64-bit, 1.8 GHz

- RAM: Available in 1 GB, 2 GB, 4 GB, and 8 GB variants

- Storage: MicroSD card (minimum 8 GB Class 10 recommended)

- OS: Raspberry Pi OS (based on Debian Linux)

- GPIO: 40-pin header, 3.3 V logic

- No ADC: Unlike Arduino/ESP, RPi has NO built-in analogue-to-digital converter. You need an external chip like the MCP3008 to read analogue sensors

- Connectivity: Gigabit Ethernet, WiFi 5 GHz, Bluetooth 5.0, 2× USB 3.0, 2× USB 2.0, 2× micro-HDMI, USB-C power

📖  Note:  The Raspberry Pi is best used as a hub or gateway in IoT systems — for example, running a database, a web server, or an MQTT broker — while ESP boards handle the actual sensor/actuator work at the edge.

## 1.3  Understanding Voltage Logic Levels

Every digital circuit uses voltage to represent data. A HIGH signal means the voltage is at or near the logic voltage. A LOW signal means the voltage is at or near 0 V (Ground). The exact voltages differ between 5 V and 3.3 V systems.

| Voltage Property | 5 V System (Arduino Uno) vs 3.3 V System (ESP32, RPi) |
| --- | --- |
| Logic HIGH output | 5.0 V |
| Valid HIGH input range | 2.5 V minimum |
| Logic LOW output | 0 V |
| Valid LOW input range | Below 1.0 V |
| Safe to receive 5 V? | Yes |
| Fix when connecting to 3.3 V device | Voltage divider or level shifter |

Voltage Divider (for connecting 5 V output to 3.3 V input)

A voltage divider uses two resistors to reduce a voltage. To step 5 V down to 3.3 V: use a 10 kΩ resistor from the 5 V signal to the output, and a 22 kΩ resistor from the output to GND. The output voltage = 5 × (22 / (10+22)) = 3.4 V — close enough to 3.3 V.

Voltage Divider formula: Vout = Vin × (R2 / (R1 + R2))
Example: Vin = 5V, R1 = 10K, R2 = 22K
Vout = 5 × (22000 / 32000) = 3.44 V  ✓ Safe for 3.3V input

Bidirectional Logic Level Shifter

When you need to convert signals in both directions (like for I2C or SPI), use a dedicated logic level shifter module such as the TXS0108E (8-channel) or BSS138-based modules. These are cheap (under $1) and handle both directions automatically.

🔑  Key Concept:  Rule of thumb: 3.3 V output driving 5 V input works fine in most cases. 5 V output driving 3.3 V input will destroy the receiver. When in doubt, always use a level shifter.

## 1.4  Power Pins Explained

Every development board has several power-related pins. Connecting the wrong voltage to the wrong pin is one of the fastest ways to destroy your board. Always look at the pinout diagram for your specific board before connecting power.

| Pin Label | Description and Safe Usage |
| --- | --- |
| VIN or RAW | Unregulated power input. Accepts 7–12 V for Arduino Uno. The onboard regulator converts this to 5 V. On ESP boards, VIN often accepts 5 V. |
| 5V | Regulated 5 V output from the onboard regulator. Do NOT feed 5 V from an external source here if you are already powering via USB or VIN. |
| 3V3 or 3.3V | Regulated 3.3 V output. The onboard LDO regulator provides this. Used to power 3.3 V sensors and modules. Max current: typically 150–500 mA. |
| GND | Ground — the common 0 V reference. Every circuit must share a common ground. Multiple GND pins on the board are all connected internally. |
| EN or RST | Enable or Reset pin. Pulling this LOW resets the microcontroller. Useful for hardware reset circuits. |
| VBUS | On some boards, this pin gives you access to the raw 5 V from the USB cable. |

⚠️  Warning:  Never connect 5 V to the 3.3 V pin. Never connect more than the rated voltage to VIN. Always double-check your wiring before applying power. Even a half-second of incorrect voltage can permanently destroy your chip.

## 1.5  Current Limits — Why They Matter

A GPIO pin can only safely source (supply) or sink (absorb) a limited amount of electrical current. Exceeding this limit permanently damages the pin — or in some cases, the entire microcontroller.

| Board | Max Per Pin / Max Total GPIO Current / Notes |
| --- | --- |
| Arduino Uno (ATmega328P) | 40 mA max / 200 mA total / Absolute max; keep under 20 mA per pin in practice |
| ESP8266 (NodeMCU) | 12 mA max / ~60 mA total / Very sensitive — use transistors for relays, buzzers |
| ESP32 | 40 mA max / 1200 mA total / Rated for 40 mA but 12 mA recommended per pin |
| Raspberry Pi 4 | 16 mA max / 50 mA total / All GPIO combined. Never drive motors directly. |

What this means in practice

- LED (20 mA typical): Safe to drive directly from GPIO with a current-limiting resistor

- Buzzer (20–40 mA): Check current draw. May need a transistor on ESP8266

- Relay coil (50–100 mA): NEVER directly from GPIO. Always use a transistor or relay module with driver

- DC motor (100 mA – 2 A): Always use a motor driver IC such as L298N or L9110S

- Servo motor (500 mA – 1 A): Use external 5 V power supply, not the board regulator

🔑  Key Concept:  Calculating current-limiting resistor for an LED: R = (Supply Voltage − LED Forward Voltage) ÷ Desired Current. Example: (5V − 2V) ÷ 0.020A = 150 Ω. Use the next standard value up: 220 Ω or 330 Ω.

## 1.6  Breadboard Wiring Guide

A solderless breadboard allows you to prototype circuits quickly without soldering. Understanding its internal connections is essential.

How the internal connections work

- Power rails (long horizontal rows at top and bottom): Each row runs the full length of the breadboard. The + rail is for your positive supply (3.3 V or 5 V). The − rail is for GND. Always connect these first.

- Component rows (short vertical groups of 5): On each half of the board, every set of 5 holes labelled A–E (or F–J) in the same row number are connected together. Inserting a component leg into hole B5 connects it to anything in A5, C5, D5, or E5.

- The centre channel: The gap running down the middle separates the two halves. This is designed for IC chips — each leg of the chip lands in a separate group on each side.

- Long vs short breadboards: Some breadboards have a break in the middle of the power rails. Connect both halves if your circuit spans the full board.

🛠️  Try It:  Exercise: Wire an LED circuit. Connect a 330 Ω resistor from the 5 V rail to one leg, then connect the other leg of the resistor to the anode (longer leg) of the LED, then connect the cathode (shorter leg) to the GND rail. Observe the LED light up without connecting to the board yet — just use the power rails directly.

## 1.7  Resistor Colour Codes

Resistors have coloured bands that encode their value and tolerance. For the most common 4-band resistors:

| Colour | Digit value / Multiplier / Tolerance |
| --- | --- |
| Black | 0 / ×1 / — |
| Brown | 1 / ×10 / ±1% |
| Red | 2 / ×100 / ±2% |
| Orange | 3 / ×1,000 / — |
| Yellow | 4 / ×10,000 / — |
| Green | 5 / ×100,000 / ±0.5% |
| Blue | 6 / ×1,000,000 / ±0.25% |
| Violet | 7 / ×10,000,000 / ±0.1% |
| Grey | 8 / ×100,000,000 / ±0.05% |
| White | 9 / — / — |
| Gold | — / ×0.1 / ±5% |
| Silver | — / ×0.01 / ±10% |

How to read a 4-band resistor: Hold the resistor with the Gold or Silver tolerance band on the right. Read bands left to right. Bands 1 and 2 are the first two digits. Band 3 is the multiplier. Band 4 is tolerance.

Example: Red – Violet – Red – Gold
Band 1 (Red)    = 2
Band 2 (Violet) = 7
Band 3 (Red)    = ×100
Band 4 (Gold)   = ±5%
Value = 27 × 100 = 2700 Ω = 2.7 kΩ  ±5%

💡  Pro Tip:  Mnemonic for the colour order: "Bad Boys Race Our Young Girls But Violet Gives Winners" = Black, Brown, Red, Orange, Yellow, Green, Blue, Violet, Grey, White.

## 1.8  Pull-up and Pull-down Resistors

This concept is fundamental to reliable digital circuits. When a GPIO pin is configured as an input and nothing is driving it to HIGH or LOW, the pin is said to be "floating." A floating pin picks up electrical noise and will randomly read HIGH or LOW — causing completely unpredictable behaviour in your code.

The solution: Pull resistors

A pull resistor connects the pin to a known voltage when nothing else is driving it, ensuring it always has a defined state.

- Pull-up resistor: A resistor (typically 10 kΩ) connected between the pin and VCC. When nothing else is connected, the pin is pulled HIGH. When a button connects the pin to GND, the pin goes LOW. This is the most common configuration — most MCUs have internal pull-ups you can enable in software.

- Pull-down resistor: A resistor connected between the pin and GND. When nothing else is connected, the pin is pulled LOW. When a button connects the pin to VCC, the pin goes HIGH.

// Enabling the internal pull-up resistor in Arduino
pinMode(2, INPUT_PULLUP);
// With INPUT_PULLUP:
//   Button NOT pressed → pin reads HIGH (pulled to VCC internally)
//   Button pressed (connects pin to GND) → pin reads LOW
// Your logic is inverted compared to what you might expect:
if (digitalRead(2) == LOW) {
  // Button is being pressed
}

🔑  Key Concept:  Always use INPUT_PULLUP (or a physical 10 kΩ pull-up resistor) for buttons and switches. Floating inputs are one of the most common causes of mysterious, hard-to-debug behaviour.

Session Summary - Microcontrollers (Arduino, ESP32) are all-in-one chips. Raspberry Pi is a full Linux computer. - ESP8266 and ESP32 use 3.3 V logic — never connect 5 V signals directly. - Each GPIO pin has a current limit. Use transistors or driver ICs for motors and relays. - Power pins: VIN accepts raw power, 5V and 3.3V pins are regulated outputs. - Pull-up resistors prevent floating inputs. Use INPUT_PULLUP on buttons. - Breadboard vertical columns are internally connected. Power rails run horizontally.

| SESSION 02 | Pins — Digital, Analogue & PWM Reading sensors, controlling outputs, and simulating analogue signals |
| --- | --- |

By the end of this session you will understand: the difference between digital and analogue signals, how to read and write GPIO pins, ADC resolution and voltage conversion, PWM and how it simulates analogue output, and pin numbering systems on all three platforms.

## 2.1  GPIO — General Purpose Input / Output

GPIO pins are the primary way your code talks to the physical world. Each pin can be configured as either an input (reading a signal) or an output (driving a signal). In Arduino's programming model, you configure this in setup() using pinMode().

| Mode | What It Does / When to Use It |
| --- | --- |
| OUTPUT | The MCU drives the pin to HIGH or LOW. Use for LEDs, relays, motor drivers, buzzers, anything you want to control. |
| INPUT | The MCU reads the voltage on the pin. Use for sensors and signals — but always add a pull resistor to avoid floating. |
| INPUT_PULLUP | Same as INPUT but enables the internal pull-up resistor. The pin reads HIGH when nothing is connected. Ideal for buttons connected to GND. |
| INPUT_PULLDOWN (ESP32) | Same as INPUT but enables the internal pull-down resistor. Pin reads LOW when nothing connected. ESP32-specific. |

// Configure and use pins
void setup() {
  pinMode(13, OUTPUT);        // LED pin as output
  pinMode(2,  INPUT_PULLUP);  // Button pin with internal pull-up
}
void loop() {
  if (digitalRead(2) == LOW) {  // Button pressed (active LOW)
    digitalWrite(13, HIGH);      // Turn LED on
  } else {
    digitalWrite(13, LOW);       // Turn LED off
  }
}

🛠️  Try It:  Wire a button between pin 2 and GND (no resistor needed — use INPUT_PULLUP). Wire an LED with a 220 Ω resistor to pin 13 and GND. Upload the code above. The LED should light when you press the button.

## 2.2  Digital Signals in Depth

A digital signal has exactly two states. This binary nature is fundamental to how computers and microcontrollers work.

- HIGH (Logic 1): The pin outputs or measures a voltage at or near the logic supply voltage. On Arduino Uno: ~5 V. On ESP32: ~3.3 V.

- LOW (Logic 0): The pin outputs or measures 0 V (GND).

- Transition: The change from LOW to HIGH is called a rising edge. The change from HIGH to LOW is called a falling edge. These edges can trigger interrupts.

Digital I/O functions

| Function | What It Does |
| --- | --- |
| pinMode(pin, mode) | Set a pin as INPUT, OUTPUT, or INPUT_PULLUP |
| digitalWrite(pin, HIGH/LOW) | Set an output pin to HIGH (VCC) or LOW (GND) |
| digitalRead(pin) | Read an input pin — returns HIGH or LOW |
| digitalPinToInterrupt(pin) | Convert a pin number to its interrupt number for attachInterrupt() |

## 2.3  Analogue Signals and the ADC

The real world is analogue — temperature, light, sound, and distance all vary smoothly across a continuous range. To read these values, microcontrollers use an Analogue-to-Digital Converter (ADC).

How ADC works

The ADC samples the voltage on the analogue pin at a moment in time and converts it to a digital number. The precision of this conversion depends on the resolution (number of bits).

- 10-bit resolution (Arduino Uno): Divides the voltage range into 2¹⁰ = 1024 steps. analogRead() returns a value from 0 to 1023.

- 12-bit resolution (ESP32): Divides into 2¹² = 4096 steps. Returns 0 to 4095. Four times the precision of Arduino.

- Reference voltage (Vref): The voltage that corresponds to the maximum reading. Arduino Uno: 5 V. ESP32: 3.3 V. The formula to convert raw ADC to voltage is: Voltage = (raw / maxRaw) × Vref.

// Arduino Uno (10-bit, 5V reference)
int raw = analogRead(A0);              // 0 to 1023
float voltage = raw * (5.0 / 1023.0); // Convert to volts
Serial.print("Voltage: "); Serial.println(voltage);
// Map to a percentage (e.g. moisture sensor)
int percent = map(raw, 0, 1023, 0, 100);
// ESP32 (12-bit, 3.3V reference)
int raw32 = analogRead(34);              // 0 to 4095
float v32  = raw32 * (3.3 / 4095.0);

| Board | ADC Pins / Resolution / Reference Voltage / Notes |
| --- | --- |
| Arduino Uno | A0–A5 (6 pins) / 10-bit (0–1023) / 5 V / Can use analogReference() to change Vref |
| Arduino Nano | A0–A7 (8 pins) / 10-bit (0–1023) / 5 V / Same as Uno |
| ESP8266 NodeMCU | A0 only! (1 pin) / 10-bit / 3.3 V max (1 V on chip, divider on board) / Very limited |
| ESP32 | ADC1: GPIO32–39 (8 pins) / 12-bit (0–4095) / 3.3 V / Prefer ADC1; ADC2 conflicts with WiFi |
| Raspberry Pi 4 | NONE built-in / N/A / N/A / Must use MCP3008 (SPI ADC) or ADS1115 (I2C ADC) |

⚠️  Warning:  On ESP32, ADC2 pins cannot be used for analogRead() while WiFi is active. Always use ADC1 pins (GPIO32–GPIO39) for analogue sensors in WiFi projects.

## 2.4  DAC — Digital to Analogue Converter

A DAC does the opposite of an ADC — it takes a digital number and outputs a true analogue voltage. This is useful for generating audio signals, variable voltage references, or smooth control signals.

- Arduino Uno / Nano: No DAC. Uses PWM as an approximation (see next section).

- Arduino Due / Zero: True 12-bit DAC on DAC0 and DAC1 pins.

- ESP32: True 8-bit DAC on GPIO25 and GPIO26. Outputs 0–3.3 V. Called with dacWrite(pin, value) where value is 0–255.

- Raspberry Pi: No built-in DAC. Use the I2S interface for audio or an external DAC chip over SPI/I2C.

// ESP32 DAC — output a true analogue voltage
dacWrite(25, 0);    // 0 V output
dacWrite(25, 128);  // ~1.65 V output (half of 3.3 V)
dacWrite(25, 255);  // ~3.3 V output

## 2.5  PWM — Pulse Width Modulation

PWM is one of the most important concepts in embedded systems. It allows a digital pin (which can only be fully ON or fully OFF) to simulate an analogue voltage by rapidly switching between HIGH and LOW states. The eye perceives a flickering LED as dimmer; a motor averages the pulses into a proportional speed.

Key PWM terminology

- Period (T): The total duration of one ON + OFF cycle, measured in milliseconds. Determined by frequency.

- Frequency (f): How many complete cycles happen per second, measured in Hz. f = 1/T. Arduino default PWM is 490 Hz on most pins.

- Duty cycle: The fraction of the period that the signal is HIGH, expressed as a percentage. 0% = always off. 100% = always on. 50% = half on, half off.

- Effective voltage: The average voltage seen by the load = Duty cycle × Logic voltage. 50% duty on 5 V system = 2.5 V effective.

// Arduino PWM — only on pins marked ~ (3,5,6,9,10,11 on Uno)
analogWrite(9, 0);    // 0% duty   = 0 V effective = LED off
analogWrite(9, 64);   // 25% duty  = ~1.25 V effective
analogWrite(9, 128);  // 50% duty  = ~2.5 V effective
analogWrite(9, 192);  // 75% duty  = ~3.75 V effective
analogWrite(9, 255);  // 100% duty = ~5 V effective = fully on
// ESP32 PWM — more flexible, any output pin
ledcSetup(0, 5000, 8);   // Channel 0, 5000 Hz, 8-bit resolution
ledcAttachPin(18, 0);    // Attach GPIO18 to channel 0
ledcWrite(0, 128);       // 50% duty cycle

⚠️  Warning:  On Arduino Uno, analogWrite() only works on pins 3, 5, 6, 9, 10, and 11 — the ones marked with a tilde (~) symbol. Using analogWrite() on any other pin will not produce PWM; it will simply behave like digitalWrite() with either HIGH or LOW.

🔑  Key Concept:  PWM does not generate a true analogue voltage. It is a digital signal switching rapidly. Most actuators (LEDs, motors, servos) respond to the average, which makes PWM an effective workaround. But sensitive analogue circuits (like audio amplifiers) may need a low-pass RC filter after the PWM pin to smooth the signal.

## 2.6  Pin Numbering — The Gotcha That Breaks Beginners

Every single platform has its own pin numbering convention, and they do not always match the numbers printed on the board. This is the most common cause of "why is nothing happening?" for beginners.

### Arduino Uno and Nano

The numbers silkscreened on the board match directly what you use in code. D2 on the board = pinMode(2, ...) in code. A0 on the board = analogRead(A0) or analogRead(0) in code. Simple.

### ESP8266 NodeMCU (The Confusing One)

The NodeMCU board labels its pins D0, D1, D2... but these do NOT correspond to GPIO numbers of the same digit. You must use a pinout chart.

| NodeMCU Label | Actual GPIO Number / Special notes |
| --- | --- |
| D0 | GPIO16 / Connected to RST for deep sleep wake-up. No interrupts. No PWM. |
| D1 | GPIO5 / Default SCL for I2C |
| D2 | GPIO4 / Default SDA for I2C |
| D3 | GPIO0 / Connected to FLASH button. Must be HIGH at boot. |
| D4 | GPIO2 / Connected to onboard LED. Must be HIGH at boot. |
| D5 | GPIO14 / Default SCK for SPI |
| D6 | GPIO12 / Default MISO for SPI |
| D7 | GPIO13 / Default MOSI for SPI |
| D8 | GPIO15 / Must be LOW at boot. Not suitable for pull-up input. |
| A0 | Analogue input only — max 3.3 V (board has divider from chip's 1 V max) |

⚠️  Warning:  On NodeMCU, GPIO0, GPIO2, and GPIO15 control boot mode. GPIO0 and GPIO2 must be HIGH at boot for normal operation. GPIO15 must be LOW. Avoid connecting sensors to these pins in ways that could pull them to the wrong level at power-on.

### ESP32

Most ESP32 breakout boards print GPIO numbers directly (GPIO2, GPIO4, etc.). Use those numbers in your code. Important restrictions:

- GPIO34, 35, 36, 39: Input only. No output, no internal pull-up or pull-down. Use only for reading sensors.

- GPIO6–11: Connected to the internal Flash memory. Do NOT use these.

- GPIO0, GPIO2: Affect boot mode. GPIO2 is also connected to the onboard LED on most DevKit boards.

### Raspberry Pi — BCM vs BOARD

Raspberry Pi has two completely different numbering systems. You must declare which one you are using at the top of every script.

import RPi.GPIO as GPIO
# Option 1: BCM numbering — use the GPIO chip numbers
# (printed as "GPIO17" on pinout diagrams)
GPIO.setmode(GPIO.BCM)
GPIO.setup(17, GPIO.OUT)   # Controls the pin labelled GPIO17
# Option 2: BOARD numbering — use the physical pin position 1–40
GPIO.setmode(GPIO.BOARD)
GPIO.setup(11, GPIO.OUT)   # Pin 11 in physical position on the header

💡  Pro Tip:  Always print out the pinout chart for your specific board and keep it on the desk during lab sessions. Search "[board name] pinout" for your exact board model — Uno, Nano, NodeMCU v3, ESP32 DevKit, RPi 4 — and use that specific chart.

Session Summary - pinMode() configures a pin; digitalWrite() sets output; digitalRead() reads input. - ADC converts analogue voltage to a number. Arduino: 10-bit (0–1023). ESP32: 12-bit (0–4095). - PWM simulates analogue by rapidly switching HIGH/LOW. Duty cycle controls the effective voltage. - ESP8266 NodeMCU board labels (D0–D8) do NOT match GPIO numbers — always use a pinout chart. - ESP32 GPIO34–39 are input-only. GPIO6–11 are reserved for internal Flash. - On Raspberry Pi, declare GPIO.setmode(GPIO.BCM) or GPIO.setmode(GPIO.BOARD) before using any pin.

| SESSION 03 | Boot, Reset & Memory What happens from power-on to your first line of code |
| --- | --- |

## 3.1  The Boot Sequence Step by Step

Every time you power on or reset a microcontroller, a precise sequence of events occurs before your code runs. Understanding this sequence helps you write more reliable programs and diagnose boot-related problems.

### Arduino Uno Boot Sequence

- Power is applied. The onboard voltage regulator brings the 5 V and 3.3 V rails to stable levels. A reset capacitor holds the RESET pin LOW while voltage stabilises.

- The bootloader — a small program pre-flashed at the factory — begins running from a protected section of Flash memory.

- The bootloader briefly listens on the USB serial port (approximately 2 seconds) for an incoming firmware upload from the IDE.

- If no upload arrives, the bootloader jumps to your application code and calls setup() once, then enters the infinite loop() cycle.

### ESP32 Boot Sequence

- First-stage bootloader runs from a small ROM inside the chip. It reads the state of GPIO0 and GPIO2 to determine boot mode.

- GPIO0 HIGH + GPIO2 HIGH = Normal boot. Loads the second-stage bootloader from Flash.

- GPIO0 LOW = Download mode. Waits for firmware upload via serial UART0. This is what happens when you click Upload in the IDE (the DTR/RTS pins on the USB chip pull GPIO0 and EN).

- Second-stage bootloader loads the main application from Flash and hands off control.

### Raspberry Pi Boot Sequence

- GPU firmware stored in the first partition of the microSD card (/boot) begins running — the CPU is actually off at this point.

- GPU firmware initialises hardware, loads the kernel (vmlinuz) and device tree into RAM.

- CPU starts, Linux kernel initialises all drivers and hardware.

- systemd (the init system) starts all configured services in parallel.

- Desktop environment or command-line login prompt appears.

🔑  Key Concept:  The 2-second pause at the beginning of every Arduino Uno sketch is the bootloader waiting. Do not put time-critical code in the first 2 seconds after boot.

## 3.2  Types of Reset and When They Happen

| Reset Type | Cause / What Happens / Retained State |
| --- | --- |
| Power-On Reset (POR) | Board powered from completely off / Full cold start, bootloader runs / Nothing retained |
| External / Manual Reset | RST button pressed or RST pin pulled LOW / Bootloader runs, then application / Nothing in SRAM |
| Software Reset | Code calls ESP.restart() or watchdog reset / Fast restart, skips some init / RTC memory retained on ESP32 |
| Watchdog Reset | Code hung for longer than WDT timeout / Automatic hardware reset / RTC memory retained on ESP32 |
| Brown-out Reset (BOR) | Supply voltage dropped below safe threshold / Resets chip to prevent data corruption / Nothing retained |
| Power-Down Reset | Deep sleep wake-up on ESP32 / Resumes at beginning of setup() / RTC memory retained |

## 3.3  Memory Types — The Most Important Thing to Understand

Running out of memory is the most common non-obvious bug for beginners. Sketches that work fine for simple programs suddenly crash when more features are added, and the cause is almost always memory exhaustion. Understanding the three types of memory prevents this.

### Flash Memory (Program Storage)

Think of Flash memory as a book that has been printed. You can read from it quickly, but you can only rewrite it a limited number of times (typically 10,000 to 100,000 erase-write cycles). Your compiled firmware is stored here permanently — it survives power-off.

| Board | Flash Size / Typical Sketch Size / Notes |
| --- | --- |
| Arduino Uno / Nano | 32 KB / 2–20 KB typical / You will see "X% of program storage space used" when compiling |
| ESP8266 | 4 MB (varies by module) / Hundreds of KB possible / Large enough for OTA updates |
| ESP32 | 4 MB (typical) / Can be up to ~1.5 MB for application / Partition table divides Flash into sections |
| Raspberry Pi | 4–256 GB microSD card / Entire Linux OS fits / Effectively unlimited for most purposes |

### SRAM — Static RAM (Working Memory)

SRAM is where your running program lives. Think of it as a whiteboard — fast to read and write, but completely erased when power is removed. Every variable you declare, every function you call, every string you create takes up SRAM.

On the Arduino Uno you have only 2 KB of SRAM. This is tiny. To put it in perspective, a single string like "Hello, World!" takes 13 bytes. One float variable takes 4 bytes. The Serial library itself uses a 64-byte buffer. SRAM fills up fast.

// How to check free SRAM on Arduino
int freeRAM() {
  extern int __heap_start, *__brkval;
  int v;
  return (int)&v - (__brkval == 0 ? (int)&__heap_start : (int)__brkval);
}
void setup() {
  Serial.begin(9600);
  Serial.print("Free SRAM: ");
  Serial.print(freeRAM());
  Serial.println(" bytes");
}
// On a fresh Uno with just Serial started: roughly 1750 bytes free
// Warning: if this drops below ~200 bytes, expect random crashes

💡  Pro Tip:  On Arduino, use the F() macro to store string literals in Flash memory instead of SRAM. Serial.println(F("This stays in Flash")); instead of Serial.println("This eats SRAM");. This single trick can save hundreds of bytes on SRAM.

### EEPROM — Electrically Erasable Programmable ROM (Persistent Storage)

EEPROM is like a small notebook that survives being switched off. It is much slower to write than SRAM, but the data persists between power cycles. Use it for settings, calibration values, and configuration that should survive a reset.

| Board | EEPROM Size / Write Cycles / Special Notes |
| --- | --- |
| Arduino Uno / Nano | 1 KB (1024 bytes) / ~100,000 writes per address / Access via #include <EEPROM.h> |
| ESP8266 | Emulated in Flash (default 512 B–4 KB) / ~100,000 cycles / Must call EEPROM.commit() to actually save |
| ESP32 | Emulated in Flash (up to 4 KB) / ~100,000 cycles / Same API as ESP8266; also has Preferences library (easier) |
| Raspberry Pi | None built-in / N/A / Use files, SQLite, or a real-time clock module's EEPROM |

// Arduino: Write and read EEPROM
#include <EEPROM.h>
void writeSettings() {
  EEPROM.write(0, 42);           // Write 1 byte at address 0
  EEPROM.write(1, 255);          // Write 1 byte at address 1
  EEPROM.put(10, 3.14f);         // Write a float (4 bytes) starting at address 10
}
void readSettings() {
  byte val  = EEPROM.read(0);    // Read 1 byte
  float f;
  EEPROM.get(10, f);             // Read float back into variable f
}
// ESP32: The Preferences library (recommended over raw EEPROM)
#include <Preferences.h>
Preferences prefs;
void saveWiFiCreds(String ssid, String pass) {
  prefs.begin("wifi", false);    // Namespace "wifi", read-write mode
  prefs.putString("ssid", ssid);
  prefs.putString("pass", pass);
  prefs.end();
}
String loadSSID() {
  prefs.begin("wifi", true);     // Read-only mode
  String s = prefs.getString("ssid", "default");
  prefs.end();
  return s;
}

## 3.4  Clock Speed and What It Means

The clock is the metronome of the microcontroller. Every instruction the CPU executes takes a certain number of clock cycles. A faster clock means more instructions per second — but also more power consumption and heat.

| Board | Clock Speed / Equivalent Instructions/sec / Notes |
| --- | --- |
| Arduino Uno | 16 MHz / ~16 million simple instructions/sec / Uses external 16 MHz crystal for accuracy |
| ESP8266 | 80 or 160 MHz / ~80–160 million/sec / Can be set in code; higher speed draws more power |
| ESP32 | Up to 240 MHz (dual core) / ~480 million/sec combined / Can be reduced for power saving: setCpuFrequencyMhz(80) |
| Raspberry Pi 4 | 1,800 MHz × 4 cores / Billions of operations/sec / Plus GPU — raw compute power dwarfs all MCUs |

## 3.5  The Watchdog Timer

The Watchdog Timer (WDT) is a hardware safety mechanism. It is an independent timer that runs constantly. Your code must periodically "reset" or "feed" the watchdog. If your code freezes, crashes, or gets stuck in an infinite loop, the watchdog timer expires and automatically resets the entire microcontroller. This is how embedded devices self-recover from software bugs.

// On ESP8266 and ESP32, the WiFi stack automatically feeds the WDT
// BUT: if you write a long blocking loop, you block the WiFi stack,
// which stops the WDT from being fed, causing a reset after ~3 seconds.
// Solution 1: Call yield() in long loops
for (int i = 0; i < 100000; i++) {
  doSomething();
  yield();     // Feeds watchdog, lets WiFi stack run
}
// Solution 2: Use millis()-based timing instead of blocking delay()
// Solution 3: Use delay(1) which internally calls yield()
// On ESP32, you can also disable the WDT (use with caution):
#include "esp_task_wdt.h"
esp_task_wdt_init(30, false);  // Set WDT timeout to 30 seconds

⚠️  Warning:  Never write code that blocks the main loop for more than a few seconds on ESP8266/ESP32 without calling yield(). Downloading a large file, waiting in a while loop for a sensor, or doing heavy computation without yield() will trigger the watchdog and reset your device.

Session Summary - Arduino bootloader waits ~2 seconds for upload before running your code. - Flash stores your program permanently. SRAM holds running variables. EEPROM stores settings across power cycles. - Arduino Uno has only 2 KB SRAM. Use the F() macro for string literals to save SRAM. - ESP32/ESP8266 EEPROM is emulated in Flash. Always call EEPROM.commit() (or use Preferences library). - The Watchdog Timer automatically resets a hung device. Feed it with yield() in long loops. - Clock speed determines how fast instructions execute. Higher speed = more power consumption.

| SESSION 04 | Code, Firmware & the IDE Writing, compiling, uploading, and organising your programs |
| --- | --- |

## 4.1  What Is Firmware?

Firmware is the compiled program that runs directly on a microcontroller. Unlike the apps on your phone or computer (which sit on top of an operating system), firmware runs on bare metal — there is nothing beneath it but the hardware itself. Understanding this pipeline from source code to running firmware is fundamental.

- You write source code in a high-level language (C++, Python, or MicroPython).

- A compiler (like avr-gcc for Arduino, or esptool for ESP) translates your source code into machine code — the specific binary instructions that the target MCU can execute.

- The compiled result is a binary file (a .hex or .bin file).

- This binary is flashed (written) into the MCU's Flash memory via a USB-to-serial connection.

- On next boot, the MCU reads these binary instructions from Flash and executes them.

🔑  Key Concept:  The IDE handles steps 2–4 automatically when you click "Upload". Under the hood it is calling the compiler (avr-gcc), linker, and programmer (avrdude) as separate tools.

## 4.2  Arduino Sketch Structure — Every Single Part

// ─── 1. PREPROCESSOR DIRECTIVES ───
// These are processed before compilation.
// #include brings in a library.
// #define creates a constant (no memory used).
#include <Wire.h>      // Include the I2C library
#define LED_PIN 13     // Constant — no RAM used, replaced at compile time
#define BAUD 115200
// ─── 2. GLOBAL VARIABLES ───
// Declared outside any function — accessible everywhere.
// Exist for the entire lifetime of the program.
// Stored in SRAM.
int counter = 0;
bool isRunning = false;
const float PI_APPROX = 3.14159; // const saves accidentally modifying it
// ─── 3. FUNCTION PROTOTYPES (optional in .ino files) ───
// Arduino IDE adds these automatically for .ino files.
// Required in .cpp files when calling a function defined below where it is used.
void blinkLED(int times, int delayMs);
// ─── 4. SETUP — runs ONCE ───
void setup() {
  // Called once after power-on or reset.
  // Put all initialisation code here.
  Serial.begin(BAUD);         // Start serial communication
  pinMode(LED_PIN, OUTPUT);   // Configure pin direction
  Wire.begin();               // Start I2C bus
  Serial.println(F("Board initialised."));
}
// ─── 5. LOOP — runs FOREVER ───
void loop() {
  // Called repeatedly, as fast as possible, with no delay between calls.
  // Your main program logic goes here.
  counter++;
  if (counter >= 10) {
    blinkLED(3, 200);
    counter = 0;
  }
}
// ─── 6. CUSTOM FUNCTIONS ───
void blinkLED(int times, int delayMs) {
  for (int i = 0; i < times; i++) {
    digitalWrite(LED_PIN, HIGH);
    delay(delayMs);
    digitalWrite(LED_PIN, LOW);
    delay(delayMs);
  }
}

## 4.3  Data Types in Embedded Systems

Choosing the right data type is more important on a microcontroller than on a PC because every byte of SRAM counts. Using a 32-bit int when a 1-byte uint8_t would suffice wastes 3 bytes — multiply that across hundreds of variables and you will run out of memory.

| Type | Size (Arduino) / Range | When to Use |
| --- | --- | --- |
| bool | 1 byte / true or false | Boolean flags, states, toggle variables |
| byte / uint8_t | 1 byte / 0 to 255 | Pin values, array indices, small counters |
| int8_t | 1 byte / −128 to 127 | Small signed values |
| int / int16_t | 2 bytes / −32,768 to 32,767 | General purpose. Default integer type. |
| unsigned int | 2 bytes / 0 to 65,535 | Positive-only counters |
| long / int32_t | 4 bytes / ±2.1 billion | millis(), large counters, timestamps |
| unsigned long | 4 bytes / 0 to ~4.3 billion | millis() return type — MUST use this |
| float | 4 bytes / ±3.4×10³⁸ (~7 sig. digits) | Sensor readings, voltage, percentages |
| double | 4 bytes on AVR, 8 on ESP32 / ±1.8×10³⁰⁸ | High precision (same as float on Uno!) |
| char | 1 byte / −128 to 127 | Single characters |
| char[] | N bytes / N−1 characters + null | Fixed strings without heap allocation |
| String | Variable (heap) / Arbitrary | Convenient but risky on small MCUs |

⚠️  Warning:  On Arduino Uno, int is 16 bits (not 32 bits like on a PC or ESP32). The return type of millis() is unsigned long. If you store millis() in a plain int, it will overflow to a negative number after about 32 seconds, breaking all your timing. Always use unsigned long for millis().

## 4.4  Header Files and #include — How Libraries Are Structured

Libraries are split into header files (.h) and source files (.cpp). The header file is the "menu" — it lists what the library can do (function names and types) without revealing the full implementation. The source file is the "kitchen" — all the actual code.

- #include <LibraryName.h>  — Use angle brackets for libraries installed in the system library folder (via Library Manager).

- #include "myfile.h"  — Use quotes for files in your own sketch folder.

- #define CONSTANT value  — Creates a compile-time constant. No memory used. The preprocessor replaces every occurrence with the value before compiling.

- #ifndef / #define / #endif  — "Include guard." Prevents a header file from being included more than once, which would cause duplicate definition errors.

// Example: myconfig.h — your own header file
#ifndef MYCONFIG_H    // Include guard: only process this file once
#define MYCONFIG_H
// WiFi credentials — keep in a separate file, never commit to GitHub
#define WIFI_SSID  "MyHomeNetwork"
#define WIFI_PASS  "mypassword123"
// Hardware pin assignments
#define DHT_PIN    4
#define LED_PIN    2
#define RELAY_PIN  5
// Configuration constants
#define MQTT_SERVER "192.168.1.100"
#define MQTT_PORT   1883
#endif   // End of include guard

💡  Pro Tip:  Separate your configuration from your main code. Create a "secrets.h" file with your WiFi password and API keys. Add "secrets.h" to your .gitignore file so it is never uploaded to GitHub. This is industry-standard practice.

## 4.5  Setting Up the Arduino IDE

### Installing Arduino IDE 2.x

- Download Arduino IDE 2.x from arduino.cc/en/software — choose the installer for your OS.

- Run the installer. On Windows it will also install required drivers.

- Install CH340 driver separately if your board uses a CH340 USB chip (common on clones) and the port does not appear.

### Adding ESP8266 Support

- Open File → Preferences (or Arduino → Preferences on Mac)

- In the "Additional boards manager URLs" field, paste: https://arduino.esp8266.com/stable/package_esp8266com_index.json

- Click OK, then open Tools → Board → Boards Manager

- Search "esp8266" and install "esp8266 by ESP8266 Community"

### Adding ESP32 Support

- Open File → Preferences again

- Add this URL (add a comma after the previous URL first): https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Open Boards Manager, search "esp32" and install "esp32 by Espressif Systems"

### Uploading Your First Sketch

- Connect your board via USB cable.

- Go to Tools → Board and select your board (e.g. "Arduino Uno" or "ESP32 Dev Module").

- Go to Tools → Port and select the port showing your board (COMx on Windows, /dev/ttyUSBx or /dev/cu.usbserial on Mac/Linux).

- Open File → Examples → Basics → Blink and click the Upload button (right arrow).

- Watch the bottom status bar — you should see "Compiling…" then "Uploading…" then "Done uploading."

- The onboard LED (pin 13) should now blink once per second.

⚠️  Warning:  If the IDE shows "avrdude: stk500_recv(): programmer is not responding" for Arduino, the board is likely on the wrong COM port, the wrong board is selected, or the USB cable is power-only (no data lines). Try different ports or a different cable.

## 4.6  PlatformIO — The Professional Alternative

PlatformIO is a VS Code extension that provides a much more powerful and professional development environment than Arduino IDE. It supports all the same boards and more, with better library management, faster compilation, and full C++ IntelliSense (code completion, error highlighting as you type).

### Setting Up PlatformIO

- Download and install Visual Studio Code from code.visualstudio.com

- Open VS Code, click the Extensions icon (four squares), search "PlatformIO IDE" and install it

- Wait for PlatformIO to install its core tools (takes 2–5 minutes on first run)

- Click the PlatformIO icon in the left sidebar, then "New Project"

- Choose your board (type to search), framework (Arduino), and a project location

### Understanding platformio.ini

Every PlatformIO project has a platformio.ini configuration file. This is equivalent to selecting Board and Port in the Arduino IDE, but more explicit and version-controlled.

; platformio.ini — the heart of every PlatformIO project
[env:esp32dev]
platform  = espressif32      ; Which chip family
board     = esp32dev         ; Specific board name
framework = arduino          ; Use Arduino framework (vs. ESP-IDF)
monitor_speed = 115200       ; Serial monitor baud rate
monitor_filters = esp32_exception_decoder  ; Decode crash reports
; Libraries are declared here — PlatformIO downloads them automatically
lib_deps =
  adafruit/DHT sensor library@^1.4.4
  adafruit/Adafruit SSD1306@^2.5.7
  knolleary/PubSubClient@^2.8.0
  bblanchon/ArduinoJson@^6.21.3
; Upload settings
upload_speed   = 921600
upload_port    = /dev/ttyUSB0  ; Or COM3 on Windows
; For OTA upload (replace serial upload with WiFi)
; upload_protocol = espota
; upload_port     = 192.168.1.100

## 4.7  MicroPython and Thonny IDE

MicroPython is a lean implementation of Python 3 designed to run directly on microcontrollers. It allows Python programmers to work with embedded hardware without learning C++. The Thonny IDE is the simplest way to use MicroPython.

### Flashing MicroPython onto an ESP32

- Install Python on your PC, then install esptool: pip install esptool

- Download the MicroPython firmware for ESP32 from micropython.org/download/esp32/

- Erase the Flash: esptool.py --port COM3 erase_flash

- Flash MicroPython: esptool.py --port COM3 --baud 460800 write_flash -z 0x1000 esp32.bin

- Open Thonny, go to Tools → Options → Interpreter, select "MicroPython (ESP32)" and the correct COM port

### MicroPython Code Structure

MicroPython does not use setup() and loop(). It uses standard Python — the file runs top to bottom. Files named main.py run automatically at boot.

# main.py — runs automatically when ESP32 boots
from machine import Pin, ADC, PWM
import time, network
# ── GPIO ──
led  = Pin(2, Pin.OUT)           # Output pin
btn  = Pin(0, Pin.IN, Pin.PULL_UP)  # Input with pull-up
# ── ADC ──
sensor = ADC(Pin(34))            # ADC on GPIO34
sensor.atten(ADC.ATTN_11DB)      # Allow full 3.3V range
raw = sensor.read()              # Returns 0–4095
# ── PWM ──
pwm = PWM(Pin(18), freq=5000)    # 5 kHz PWM on GPIO18
pwm.duty(512)                    # 50% duty (0–1023 on ESP32 MicroPython)
# ── Main loop ──
while True:
    led.on()
    time.sleep(0.5)
    led.off()
    time.sleep(0.5)
    print("Button:", btn.value())

Session Summary - Firmware pipeline: source code → compiler → binary (.hex/.bin) → flash → runs on MCU. - Every Arduino sketch needs setup() (runs once) and loop() (runs forever). - Use the F() macro for string literals to save SRAM on Arduino. - Use unsigned long for millis() — int overflows in 32 seconds. - Put WiFi passwords and secrets in a separate header file and add it to .gitignore. - PlatformIO manages libraries automatically via platformio.ini — better than Arduino IDE for larger projects. - MicroPython lets you use Python on ESP32/ESP8266. Files named main.py run at boot.

| SESSION 05 | Serial, Monitor, Plotter & Debugging Communicating with your board and finding bugs systematically |
| --- | --- |

## 5.1  UART Serial Communication

UART (Universal Asynchronous Receiver-Transmitter) is the simplest and most universal form of serial communication. It is how your board sends text to your computer for debugging, and how it communicates with GPS modules, Bluetooth adapters, and many other peripherals.

How UART works

UART transmits data one bit at a time on a single wire. Both sides must agree on the transmission speed (baud rate) before communication begins. There is no shared clock signal — each side uses its own internal clock at the agreed rate. This is what "asynchronous" means.

- TX (Transmit): The pin that sends data out. Always cross-connect TX to RX of the other device.

- RX (Receive): The pin that receives data. Cross-connect to TX of the other device.

- GND: Both devices must share a common ground reference. Without this, voltages have no common reference and communication fails.

- Baud rate: Bits per second. Common rates: 9600, 19200, 38400, 57600, 115200. Both sides MUST use the same rate. Mismatch shows as garbage characters.

| Board | Hardware UART ports / Pins / Max reliable baud rate |
| --- | --- |
| Arduino Uno | 1 (shared with USB) / TX=1, RX=0 / 115200 on hardware UART |
| Arduino Mega | 4 hardware UARTs / Serial, Serial1, Serial2, Serial3 / 115200+ |
| ESP8266 | 2 hardware UARTs / UART0: GPIO1/3, UART1: GPIO2 (TX only) / 921600 |
| ESP32 | 3 hardware UARTs / UART0–2, fully remappable to any pin / 921600+ |
| Raspberry Pi | 1 primary UART / GPIO14(TX) GPIO15(RX) / Standard Linux rates |

## 5.2  The Serial Monitor — Your Best Debugging Tool

The Serial Monitor is a text terminal in the IDE that shows everything your board prints, and lets you type commands back to the board. It is the first tool you should reach for when debugging.

// ── Sending data from board to PC ──
Serial.begin(115200);          // Start UART at 115200 baud
Serial.print("Count: ");       // Print without newline
Serial.println(count);         // Print with newline (\n)
Serial.print(3.14159, 4);      // Print float with 4 decimal places
Serial.print(255, BIN);        // Print in binary: 11111111
Serial.print(255, HEX);        // Print in hex: FF
Serial.print(255, OCT);        // Print in octal: 377
// ── Receiving data from PC to board ──
if (Serial.available() > 0) {  // Bytes waiting in receive buffer?
  String cmd = Serial.readStringUntil('\n'); // Read until newline
  cmd.trim();                  // Remove trailing whitespace/newline
  if (cmd == "ON") {
    digitalWrite(LED_PIN, HIGH);
    Serial.println("LED turned ON");
  }
  if (cmd == "OFF") {
    digitalWrite(LED_PIN, LOW);
    Serial.println("LED turned OFF");
  }
}

⚠️  Warning:  In the Arduino IDE Serial Monitor, make sure the baud rate dropdown at the bottom matches the number in Serial.begin(). A mismatch produces garbled output that looks like "?????" or random symbols. Also make sure "Newline" is selected in the line-ending dropdown when sending commands.

## 5.3  The Serial Plotter — Visualising Data in Real Time

The Serial Plotter automatically graphs numerical data streaming from your board. Open it from Tools → Serial Plotter. It is perfect for visualising sensor readings, PID loops, and signal patterns without any extra software.

// ── Single value plot ──
int raw = analogRead(A0);
Serial.println(raw);    // Just print a number and newline — Plotter graphs it
delay(20);              // ~50 samples per second
// ── Multiple traces (Arduino IDE 2.x label format) ──
void loop() {
  int pot   = analogRead(A0);
  int light = analogRead(A1);
  int temp  = analogRead(A2);
  // Label:value format — each trace gets a different colour
  Serial.print("Potentiometer:"); Serial.print(pot);
  Serial.print(",Light:");        Serial.print(light);
  Serial.print(",Temperature:");  Serial.println(temp);
  delay(50);   // 20 Hz sample rate
}

🔑  Key Concept:  The Serial Plotter and Serial Monitor cannot be open at the same time. Close the Plotter before opening the Monitor and vice versa. Also remember that Serial.println() is what ends each data point — missing a println means the Plotter does not know when one sample ends and the next begins.

## 5.4  Debugging Strategy — A Systematic Approach

Random guessing is the slowest way to debug. Use a systematic process every time something does not work.

- Test hardware independently first: Upload Blink (File → Examples → Basics → Blink). If pin 13 blinks, your board is working and uploading correctly. If not, the problem is hardware or IDE setup, not your sketch.

- Add Serial.println() checkpoints: Print the value of every important variable at each key step in your logic. Do not guess what a variable contains — print it and see.

- Check wiring before blaming code: The majority of problems for beginners are wiring errors. Reseat every jumper wire, check polarity, confirm GND is connected. Use a multimeter to measure actual voltages.

- Isolate the problem: Comment out sections of your code until the problem disappears. The last thing you commented out is the culprit. This is called binary search debugging.

- Test libraries with their examples: Before integrating a sensor library into your main sketch, test it with the library's own example sketch (File → Examples → [Library Name]). If the example does not work, the problem is wiring or the library, not your code.

- Read the error message carefully: Arduino error messages appear at the bottom of the IDE. The line number and error type are there if you read them. "was not declared in this scope" = typo or missing include. "expected ';' before" = syntax error on the previous line.

- Search the exact error: Copy the error message and paste it into a search engine. Every common Arduino error has been discussed on forums hundreds of times.

## 5.5  millis() vs delay() — Why Non-Blocking Timing Matters

This is one of the most important concepts for writing professional embedded code. delay() is a blocking function — while it waits, your processor is completely frozen. Nothing else runs. No buttons can be read, no serial data processed, no WiFi packets handled.

millis() returns the number of milliseconds since the board powered on. Use it like a stopwatch to check if enough time has passed without stopping anything else.

// ── BLOCKING approach (BAD for real projects) ──
void loop() {
  digitalWrite(LED, HIGH);
  delay(500);             // CPU frozen — cannot read buttons
  digitalWrite(LED, LOW);
  delay(500);             // CPU frozen again
}
// ── NON-BLOCKING approach (CORRECT) ──
unsigned long previousMillis = 0;   // Must be unsigned long!
const long interval = 500;
bool ledState = false;
void loop() {
  unsigned long currentMillis = millis();
  // Check if it is time to toggle the LED
  if (currentMillis - previousMillis >= interval) {
    previousMillis = currentMillis;  // Save the timestamp
    ledState = !ledState;            // Toggle
    digitalWrite(LED, ledState);
  }
  // These run at FULL SPEED, not blocked by LED timing:
  checkButton();
  readSensor();
  updateDisplay();
  handleWiFiClient();
}

💡  Pro Tip:  Think of millis() like checking a wall clock. You glance at the clock, see if enough time has passed, and if not, you carry on doing other things and check again on the next loop. delay() is like staring at the clock and doing nothing until the time arrives.

## 5.6  Interrupts and Interrupt Service Routines (ISRs)

An interrupt is a hardware mechanism that immediately pauses whatever the CPU is doing, jumps to a special handler function (the ISR), and then resumes normal execution. Interrupts allow your code to respond to events instantly, without polling.

When to use interrupts

- Button presses that must never be missed, regardless of what else is running

- Rotary encoder reading (the pulses are too fast to poll reliably)

- Timing-critical signals like RC receiver pulses

- Sensor data-ready signals (when a sensor tells you it has new data)

ISR rules — these are strictly enforced

- Keep ISRs extremely short: An ISR should ideally just set a flag variable and return. Anything longer risks missing other interrupts.

- No delay() inside an ISR: delay() relies on a timer interrupt that is blocked while your ISR runs. It will freeze forever.

- No Serial.print() inside an ISR: Serial communication uses interrupts internally. Calling it from an ISR causes a deadlock.

- No complex operations: No memory allocation, no I2C/SPI calls, no WiFi calls.

- Declare shared variables as volatile: The volatile keyword tells the compiler that this variable can change at any time (because an ISR can modify it). Without volatile, the compiler might optimise away reads of the variable.

// ── Interrupt example: count button presses with 100% accuracy ──
volatile int pressCount = 0;     // volatile: shared with ISR
volatile bool pressed = false;   // flag set by ISR, cleared in loop()
// The ISR — MUST be as short as possible
void IRAM_ATTR onButtonPress() { // IRAM_ATTR ensures ISR is in fast RAM on ESP32
  pressCount++;
  pressed = true;
}
void setup() {
  Serial.begin(115200);
  pinMode(2, INPUT_PULLUP);
  // attachInterrupt(pin, function, mode)
  // Mode: RISING, FALLING, CHANGE, LOW, HIGH
  attachInterrupt(digitalPinToInterrupt(2), onButtonPress, FALLING);
}
void loop() {
  if (pressed) {
    pressed = false;        // Clear the flag first (before printing)
    Serial.print("Press count: ");
    Serial.println(pressCount);
  }
}

## 5.7  Debouncing Buttons

Mechanical switches are not perfect. When you press or release a button, the metal contacts bounce — they make and break contact 5 to 50 times in the first 20–50 ms before settling. To a microcontroller running at 16 MHz, this looks like dozens of button presses in milliseconds.

// ── Software debouncing ──
const int BUTTON_PIN  = 2;
const int DEBOUNCE_MS = 50;    // 50 ms is usually enough
int          lastReading   = HIGH;
int          buttonState   = HIGH;   // Debounced, stable state
unsigned long lastDebounce = 0;
void loop() {
  int reading = digitalRead(BUTTON_PIN);
  if (reading != lastReading) {
    lastDebounce = millis();    // Start timing from when state changed
  }
  if ((millis() - lastDebounce) > DEBOUNCE_MS) {
    // Signal has been stable for 50 ms — it is real
    if (reading != buttonState) {
      buttonState = reading;
      if (buttonState == LOW) {  // Active LOW with INPUT_PULLUP
        Serial.println("Button pressed — genuine, not a bounce!");
      }
    }
  }
  lastReading = reading;
}

🔑  Key Concept:  Hardware debouncing (an RC circuit: 100 Ω resistor + 100 nF capacitor on the button line) is more reliable than software debouncing, but costs more in component count. For most projects, the software method shown above is perfectly adequate.

Session Summary - UART requires matching baud rates on both sides. TX of device A connects to RX of device B. - Serial Monitor: Serial.begin() baud rate must match the IDE dropdown setting. - Serial Plotter: print numbers with Serial.println() for one trace; use "Label:value," format for multiple traces. - Debug systematically: test hardware → add print statements → isolate → check wiring → read the error. - Never use delay() when multiple things need to happen concurrently. Use millis() instead. - ISR rules: short, no delay(), no Serial, use volatile for shared variables. - Debounce buttons with a 50 ms timing window to filter out mechanical bounce.

| SESSION 06 | Communication Protocols I2C, SPI, UART, WiFi, MQTT, and HTTP for IoT |
| --- | --- |

## 6.1  Why Protocols Exist

A protocol is a set of agreed-upon rules for how two devices exchange data. Without agreed rules, one device would transmit 1s and 0s and the other would have no idea how to interpret them. Different protocols make different trade-offs between speed, pin count, distance, and simplicity.

| Protocol | Wires / Speed / Devices / Best For |
| --- | --- |
| UART | TX + RX + GND / Up to 1 Mbps / Point-to-point only / Debug output, GPS, Bluetooth serial, display modules |
| I2C | SDA + SCL + GND / 100–400 Kbps / Up to 127 on one bus / Sensors, displays, RTCs, all on just 2 wires |
| SPI | MOSI+MISO+SCK+SS+GND / 1–80 MHz / Multiple (one SS each) / SD cards, TFT displays, fast ADCs |
| 1-Wire | 1 data + GND / ~16 Kbps / Multiple (unique IDs) / DS18B20 temperature sensors |
| WiFi (TCP) | Wireless / Up to 150 Mbps / Network-wide / HTTP REST APIs, web servers, OTA updates |
| MQTT (WiFi) | Wireless (via broker) / Low overhead / Thousands / IoT sensor networks, home automation |

## 6.2  I2C in Detail

I2C (Inter-Integrated Circuit, pronounced "eye-squared-see" or "eye-two-see") is the most popular protocol for connecting sensors and displays because it only needs two wires for any number of devices.

How I2C works

One device is the master (your MCU) and the rest are slaves (sensors, displays, etc.). The master controls all communication — it initiates every transaction. The master sends the 7-bit address of the slave it wants to talk to; only that slave responds. The SDA line carries data. The SCL line carries the clock signal generated by the master.

Pull-up resistors — mandatory

Both SDA and SCL lines MUST have pull-up resistors to VCC (3.3 V or 5 V depending on your system). Without them, the lines can never reach HIGH reliably. Typical value: 4.7 kΩ. Many breakout modules include these already.

| Board | Default SDA Pin / Default SCL Pin / Notes |
| --- | --- |
| Arduino Uno / Nano | A4 (pin 18) / A5 (pin 19) / Also available on dedicated SDA/SCL header on newer boards |
| Arduino Mega | Pin 20 / Pin 21 / Different location than Uno — check your board |
| ESP8266 NodeMCU | D2 (GPIO4) / D1 (GPIO5) / Can remap with Wire.begin(sda, scl) |
| ESP32 | GPIO21 / GPIO22 / Can remap: Wire.begin(sda, scl) with any valid GPIOs |
| Raspberry Pi | GPIO2 / Pin 3 (Physical) / GPIO3 / Pin 5 (Physical) / Enable via raspi-config first |

// ── I2C Scanner — run this to find your device addresses ──
#include <Wire.h>
void setup() {
  Wire.begin();
  Serial.begin(115200);
  Serial.println("Scanning I2C bus...");
  int devicesFound = 0;
  for (byte addr = 1; addr < 127; addr++) {
    Wire.beginTransmission(addr);
    byte err = Wire.endTransmission();
    if (err == 0) {
      Serial.print("  Device found at address 0x");
      if (addr < 16) Serial.print("0"); // Leading zero
      Serial.println(addr, HEX);
      devicesFound++;
    }
  }
  if (devicesFound == 0)
    Serial.println("No devices found. Check wiring and pull-up resistors.");
}
void loop() {}

Common I2C device addresses

| I2C Address | Device / Notes |
| --- | --- |
| 0x3C or 0x3D | SSD1306 OLED display (128×64 or 128×32) / Address set by ADDR pin |
| 0x76 or 0x77 | BME280 or BMP280 pressure/temperature sensor / Address set by SDO pin |
| 0x68 or 0x69 | DS3231 RTC or MPU6050 gyro/accelerometer / Address set by AD0 pin |
| 0x27 or 0x3F | PCF8574 I2C LCD backpack / Address set by A0–A2 pins |
| 0x48–0x4B | ADS1115 16-bit external ADC / Address set by ADDR pin |
| 0x40 | INA219 current sensor or PCA9685 PWM expander |

## 6.3  SPI in Detail

SPI (Serial Peripheral Interface) is faster than I2C because it uses a dedicated clock line and dedicated data lines for each direction. It is full-duplex — it can send and receive simultaneously. The trade-off is that it needs more wires.

- MOSI (Master Out Slave In): Data from master to slave. This pin is an OUTPUT on the MCU.

- MISO (Master In Slave Out): Data from slave to master. This pin is an INPUT on the MCU.

- SCK (Clock): Clock signal generated by the master. Controls timing.

- SS / CS (Slave Select / Chip Select): One pin per device. Pull it LOW to activate a specific slave, then HIGH when done. You choose which GPIO to use for SS.

// ── SPI with SD card ──
#include <SPI.h>
#include <SD.h>
const int CS_PIN = 10;   // Chip select for SD card
void setup() {
  Serial.begin(115200);
  if (!SD.begin(CS_PIN)) {
    Serial.println("SD card init failed! Check wiring.");
    return;
  }
  // Write a file
  File f = SD.open("log.txt", FILE_WRITE);
  if (f) {
    f.println("Temperature, 24.5, Humidity, 60");
    f.close();
    Serial.println("Data logged successfully.");
  }
}

## 6.4  WiFi on ESP8266 and ESP32

### Station Mode — Connect to Your Home Network

Station mode (STA) connects the ESP board to an existing WiFi network, just like your phone or laptop. The board gets an IP address from the router and can access the internet or local network.

#include <WiFi.h>         // ESP32
// #include <ESP8266WiFi.h>  // ESP8266 — change this line for ESP8266
const char* SSID = "YourNetworkName";
const char* PASS = "YourPassword";
void setup() {
  Serial.begin(115200);
  WiFi.begin(SSID, PASS);
  Serial.print("Connecting");
  while (WiFi.status() != WL_CONNECTED) {
    delay(500);
    Serial.print(".");
  }
  Serial.println();
  Serial.print("Connected! IP address: ");
  Serial.println(WiFi.localIP());
  Serial.print("Signal strength (RSSI): ");
  Serial.print(WiFi.RSSI());
  Serial.println(" dBm");
}

### Access Point Mode — Create Your Own Network

In AP mode, the ESP creates its own WiFi network. Other devices (your phone, laptop) can connect to it directly. This is useful for configuration interfaces or in areas without existing WiFi.

WiFi.softAP("ESP32-Config", "setup1234");  // SSID, password (min 8 chars)
Serial.print("AP IP: ");
Serial.println(WiFi.softAPIP());  // Typically 192.168.4.1
Serial.print("Connected stations: ");
Serial.println(WiFi.softAPgetStationNum());

## 6.5  MQTT — The Language of IoT

MQTT (Message Queuing Telemetry Transport) is a lightweight messaging protocol designed specifically for IoT devices. It is the most widely used IoT protocol because it works well even on slow or unreliable networks, and because it decouples senders from receivers through a publish/subscribe pattern.

Core concepts

- Broker: A server that receives all messages and routes them to subscribers. Think of it as a post office. Popular brokers: Eclipse Mosquitto (self-hosted), HiveMQ Cloud, EMQX. For testing: test.mosquitto.org is free and public.

- Topic: A hierarchical string that categorises messages, like a folder path. Example: "home/livingroom/temperature". Use forward slashes as separators.

- Publish: Send a message to a topic. Any client connected to the broker can publish to any topic.

- Subscribe: Register interest in a topic. When any client publishes to that topic, the broker forwards the message to all subscribers.

- Wildcard +: Matches one level. "home/+/temperature" matches "home/bedroom/temperature" and "home/kitchen/temperature".

- Wildcard #: Matches all remaining levels. "home/#" matches everything under "home/".

- QoS 0 / 1 / 2: Quality of Service. 0 = fire and forget (no guarantee). 1 = at least once (may duplicate). 2 = exactly once (slowest but guaranteed). Use QoS 1 for most IoT sensor data.

- Retained messages: If you publish with retain=true, the broker stores the last message. Any new subscriber immediately receives this message — useful for sensor state that should always be available.

// ── Full MQTT example: ESP32 temperature publisher ──
#include <WiFi.h>
#include <PubSubClient.h>
#include <DHT.h>
#include "secrets.h"   // Contains WIFI_SSID, WIFI_PASS
#define MQTT_SERVER  "test.mosquitto.org"
#define MQTT_PORT    1883
#define CLIENT_ID    "esp32-living-room"
#define PUB_TOPIC    "home/livingroom/temperature"
#define SUB_TOPIC    "home/livingroom/led"
DHT dht(4, DHT22);
WiFiClient wifi;
PubSubClient mqtt(wifi);
// Called when a subscribed topic receives a message
void onMessage(char* topic, byte* payload, unsigned int length) {
  String msg = "";
  for (int i = 0; i < length; i++) msg += (char)payload[i];
  Serial.print("Received ["); Serial.print(topic);
  Serial.print("]: "); Serial.println(msg);
  if (msg == "1") digitalWrite(2, HIGH);
  if (msg == "0") digitalWrite(2, LOW);
}
void reconnect() {
  while (!mqtt.connected()) {
    Serial.print("Connecting to MQTT...");
    if (mqtt.connect(CLIENT_ID)) {
      Serial.println("connected!");
      mqtt.subscribe(SUB_TOPIC);   // Subscribe after connect
    } else {
      Serial.print("failed, rc="); Serial.println(mqtt.state());
      delay(5000);
    }
  }
}
void setup() {
  Serial.begin(115200); dht.begin();
  WiFi.begin(WIFI_SSID, WIFI_PASS);
  while (WiFi.status() != WL_CONNECTED) delay(500);
  mqtt.setServer(MQTT_SERVER, MQTT_PORT);
  mqtt.setCallback(onMessage);
}
unsigned long lastPublish = 0;
void loop() {
  if (!mqtt.connected()) reconnect();
  mqtt.loop();   // MUST call every loop iteration
  if (millis() - lastPublish > 30000) {  // Every 30 seconds
    lastPublish = millis();
    float t = dht.readTemperature();
    if (!isnan(t)) {
      String payload = String(t, 1);   // One decimal place
      mqtt.publish(PUB_TOPIC, payload.c_str());
      Serial.print("Published: "); Serial.println(payload);
    }
  }
}

## 6.6  HTTP REST API Calls

The ESP32 and ESP8266 can act as HTTP clients — fetching data from the internet or posting data to cloud services — just like a web browser does, but in your code.

// ── GET request — fetch weather data ──
#include <WiFi.h>
#include <HTTPClient.h>
#include <ArduinoJson.h>
void fetchWeather() {
  HTTPClient http;
  // Open-Meteo: free, no API key needed
  String url = "https://api.open-meteo.com/v1/forecast"
               "?latitude=23.81&longitude=90.41"
               "&current_weather=true";
  http.begin(url);
  int responseCode = http.GET();
  if (responseCode == 200) {
    String body = http.getString();
    // Parse JSON
    DynamicJsonDocument doc(1024);
    deserializeJson(doc, body);
    float temp  = doc["current_weather"]["temperature"];
    float wind  = doc["current_weather"]["windspeed"];
    Serial.print("Temperature: "); Serial.println(temp);
    Serial.print("Wind speed:  "); Serial.println(wind);
  } else {
    Serial.print("HTTP error: "); Serial.println(responseCode);
  }
  http.end();
}

💡  Pro Tip:  For JSON parsing, always use the ArduinoJson library (arduinojson.org). Use the ArduinoJson Assistant at arduinojson.org/v6/assistant to automatically generate parsing code — just paste your JSON response and it creates the code for you.

Session Summary - I2C needs only 2 wires for up to 127 devices. Each device has a unique 7-bit address. Requires 4.7 kΩ pull-ups. - SPI is faster than I2C but needs more wires (MOSI, MISO, SCK, and one SS pin per device). - Run the I2C Scanner sketch before using any I2C device — it tells you the address instantly. - WiFi Station mode connects to existing network. AP mode creates its own network. - MQTT uses a broker to route publish/subscribe messages. Always call mqtt.loop() in your main loop. - ArduinoJson library makes parsing JSON responses from APIs straightforward.

| SESSION 07 | Interfaces, Displays & Sensors Wiring and programming real-world components |
| --- | --- |

## 7.1  Sensor Wiring Reference

| Sensor | Wiring Summary | Library and Code Snippet |
| --- | --- | --- |
| DHT22 (Temp + Humidity) | VCC → 3.3V or 5V Data → any GPIO (e.g. GPIO4) GND → GND 10K pull-up on Data pin | #include <DHT.h> DHT dht(4, DHT22); dht.begin(); float t = dht.readTemperature(); float h = dht.readHumidity(); |
| DS18B20 (Waterproof Temp) | Red → VCC (3.3 or 5V) Yellow/White → Data pin Black → GND 4.7K pull-up on Data | #include <OneWire.h> #include <DallasTemperature.h> OneWire ow(4); DallasTemperature s(&ow); s.begin(); s.requestTemperatures(); float t = s.getTempCByIndex(0); |
| HC-SR04 (Ultrasonic) | VCC → 5V only Trig → any digital GPIO Echo → GPIO (use divider to 3.3V for ESP) GND → GND | digitalWrite(trig, HIGH); delayMicroseconds(10); digitalWrite(trig, LOW); long dur = pulseIn(echo, HIGH); float cm = dur * 0.034 / 2; |
| PIR (Motion) | VCC → 5V OUT → any digital GPIO GND → GND Adjust sensitivity pot on module | int motion = digitalRead(pirPin); // HIGH = motion detected // Adjust delay pot for hold time |
| BME280 (Pressure/Temp/Humidity) | VCC → 3.3V SDA → SDA pin SCL → SCL pin GND → GND Address: 0x76 or 0x77 | #include <Adafruit_BME280.h> Adafruit_BME280 bme; bme.begin(0x76); float t = bme.readTemperature(); float p = bme.readPressure()/100.0F; float h = bme.readHumidity(); |
| MFRC522 (RFID) | VCC → 3.3V ONLY (not 5V) SDA (SS) → GPIO10 SCK → GPIO13 MOSI → GPIO11 MISO → GPIO12 RST → GPIO9 | #include <MFRC522.h> MFRC522 rfid(SS_PIN, RST_PIN); SPI.begin(); rfid.PCD_Init(); if (rfid.PICC_IsNewCardPresent()) rfid.PICC_ReadCardSerial(); |
| MPU6050 (Gyro + Accel) | VCC → 3.3V or 5V SDA → SDA SCL → SCL GND → GND AD0 → GND for address 0x68 | #include <MPU6050_light.h> MPU6050 mpu(Wire); mpu.begin(); mpu.calcGyroOffsets(); mpu.update(); float ax = mpu.getAngleX(); |

## 7.2  OLED Display — Complete Guide

The 0.96-inch SSD1306 OLED display is the most popular display module for Arduino and ESP projects. It is 128 pixels wide and 64 pixels tall, monochrome (white, yellow, or blue pixels), connects via I2C, and is extremely visible even in bright light.

#include <Wire.h>
#include <Adafruit_GFX.h>      // Core graphics library
#include <Adafruit_SSD1306.h>  // Display driver
#define SCREEN_W   128
#define SCREEN_H    64
#define OLED_ADDR  0x3C
Adafruit_SSD1306 display(SCREEN_W, SCREEN_H, &Wire, -1);
// -1 = no reset pin
void setup() {
  if (!display.begin(SSD1306_SWITCHCAPVCC, OLED_ADDR)) {
    Serial.println("OLED not found! Check I2C address and wiring.");
    while (1);
  }
  display.clearDisplay();
  // ── Text ──
  display.setTextSize(1);              // 1=small, 2=medium, 3=large
  display.setTextColor(SSD1306_WHITE);
  display.setCursor(0, 0);            // (column, row in pixels)
  display.println("Hello, World!");
  // ── Shapes ──
  display.drawLine(0, 16, 127, 16, SSD1306_WHITE); // Horizontal line
  display.drawRect(10, 20, 50, 30, SSD1306_WHITE); // Rectangle outline
  display.fillCircle(100, 35, 15, SSD1306_WHITE);  // Filled circle
  display.display(); // ALWAYS call this — nothing shows without it!
}
// ── Live sensor display ──
void showSensorData(float temp, float humi) {
  display.clearDisplay();
  display.setTextSize(1); display.setCursor(0, 0);
  display.println("Weather Station");
  display.drawLine(0, 10, 127, 10, SSD1306_WHITE);
  display.setTextSize(2); display.setCursor(0, 15);
  display.print(temp, 1); display.println(" C");
  display.setCursor(0, 40);
  display.print(humi, 0); display.println(" %");
  display.display();
}

📖  Note:  Always call display.clearDisplay() before drawing new content, then draw everything, then call display.display() once at the end. Calling display.display() multiple times per frame causes flickering.

## 7.3  Servo Motor Control

A servo motor is a DC motor with a built-in gearbox, position sensor, and feedback controller. You simply tell it what angle to go to (0–180 degrees for a standard servo) and it moves there and holds. The control signal is a 50 Hz PWM pulse where width encodes the desired angle.

#include <Servo.h>
Servo myServo;
void setup() {
  myServo.attach(9);     // Attach to PWM pin 9 (or any pin on ESP32)
  myServo.write(90);     // Start at centre position
}
void loop() {
  // Sweep from 0 to 180 degrees
  for (int pos = 0; pos <= 180; pos++) {
    myServo.write(pos);
    delay(15);  // 15 ms per degree for smooth movement
  }
  // Sweep back from 180 to 0
  for (int pos = 180; pos >= 0; pos--) {
    myServo.write(pos);
    delay(15);
  }
}
// Control from potentiometer
void loop() {
  int raw   = analogRead(A0);
  int angle = map(raw, 0, 1023, 0, 180);
  myServo.write(angle);
  delay(20);
}

⚠️  Warning:  Servo motors draw much more current than a GPIO pin can supply. Power the servo from an external 5 V supply (or a dedicated servo power bus), not from the Arduino 5V pin. Connect the servo's GND to the same GND as the Arduino. Failing to do this causes voltage drops, resets, and jittery servo behavior.

## 7.4  DC Motor Control with L298N

The L298N is a dual H-bridge motor driver that lets you control two DC motors independently — controlling both speed (via PWM) and direction (via direction pins). It accepts 5–35 V motor supply and has a built-in 5 V regulator (up to 500 mA) that can power your Arduino.

// L298N wiring for Motor A:
// IN1, IN2 → direction control (any 2 digital pins)
// ENA      → speed control (a PWM pin)
// OUT1, OUT2 → connect to motor terminals
const int IN1 = 7;
const int IN2 = 8;
const int ENA = 9;  // Must be a PWM pin!
void motorForward(int speed) {  // speed: 0–255
  digitalWrite(IN1, HIGH);
  digitalWrite(IN2, LOW);
  analogWrite(ENA, speed);
}
void motorBackward(int speed) {
  digitalWrite(IN1, LOW);
  digitalWrite(IN2, HIGH);
  analogWrite(ENA, speed);
}
void motorBrake() {
  digitalWrite(IN1, HIGH);
  digitalWrite(IN2, HIGH);
  analogWrite(ENA, 255);  // Active brake
}
void motorCoast() {
  analogWrite(ENA, 0);    // Cut power — motor coasts to stop
}
void setup() {
  pinMode(IN1, OUTPUT); pinMode(IN2, OUTPUT); pinMode(ENA, OUTPUT);
}

## 7.5  Relay Module

A relay is an electromechanical switch — a small electromagnet that physically opens or closes contacts. A relay module includes the relay, a driver transistor, and an optocoupler for safe isolation between your 3.3/5 V circuit and the load (which might be 230 V AC mains).

| Pin on Relay Module | Connect To / Notes |
| --- | --- |
| VCC | 5 V from board (or external 5 V supply for large relays) |
| GND | GND of board |
| IN (or Signal) | Any GPIO pin. Most modules are ACTIVE LOW — writing LOW turns relay ON |
| COM (Common) | One side of your load power (e.g. one wire of mains or battery positive) |
| NO (Normally Open) | Load connection — circuit is OPEN when relay is OFF, CLOSED when relay is ON |
| NC (Normally Closed) | Load connection — circuit is CLOSED when relay is OFF, OPEN when relay is ON |

// Most relay modules are ACTIVE LOW:
// digitalWrite(relayPin, LOW)  = relay ON  (coil energised)
// digitalWrite(relayPin, HIGH) = relay OFF (coil de-energised)
const int RELAY_PIN = 5;
void setup() {
  pinMode(RELAY_PIN, OUTPUT);
  digitalWrite(RELAY_PIN, HIGH);  // Start with relay OFF
}
void loop() {
  digitalWrite(RELAY_PIN, LOW);   // Turn relay ON
  delay(5000);                    // 5 seconds on
  digitalWrite(RELAY_PIN, HIGH);  // Turn relay OFF
  delay(5000);                    // 5 seconds off
}

⚠️  Warning:  NEVER touch the COM, NO, or NC terminals while they are connected to mains voltage (230 V AC). Mains voltage is lethal. Always completely disconnect the mains power before making any connections. If you are unsure about working with mains, use a low-voltage DC load (battery-powered LED strip, DC fan, etc.) for learning.

Session Summary - DHT22 needs a 10K pull-up on data pin. Call dht.readTemperature() after dht.begin(). - Always run I2C scanner first to confirm device address before coding. - OLED: clearDisplay() → draw everything → display() once. Never skip display.display(). - Servo power from external 5 V supply — not from the Arduino 5V pin. - L298N: IN1/IN2 control direction. ENA PWM controls speed. - Relay modules are usually active LOW. HIGH = relay OFF. LOW = relay ON. - MFRC522 RFID must use 3.3 V — connecting 5 V will destroy it.

| SESSION 08 | Platform-Specific & Going Further OTA, deep sleep, Linux on RPi, and your first real project |
| --- | --- |

## 8.1  OTA Updates on ESP32/ESP8266

OTA (Over-The-Air) updates allow you to upload new firmware to an ESP board over WiFi, with no USB cable. This is essential for deployed devices — imagine your sensor is in a ceiling, or outdoors, or in a location without easy USB access.

// Complete OTA-ready template — include this in ALL ESP projects
#include <WiFi.h>
#include <ArduinoOTA.h>
#include "secrets.h"
void setupOTA() {
  ArduinoOTA.setHostname("my-esp32-sensor");  // Name shown in IDE
  ArduinoOTA.setPassword("ota_secret");        // Protect from others
  ArduinoOTA.onStart([]() {
    Serial.println("OTA update starting...");
  });
  ArduinoOTA.onEnd([]() {
    Serial.println("\nOTA done! Rebooting.");
  });
  ArduinoOTA.onProgress([](unsigned int prog, unsigned int total) {
    Serial.printf("OTA Progress: %u%%\r", (prog / (total / 100)));
  });
  ArduinoOTA.onError([](ota_error_t err) {
    Serial.printf("OTA Error [%u]\n", err);
  });
  ArduinoOTA.begin();
  Serial.println("OTA ready on: " + WiFi.localIP().toString());
}
void setup() {
  Serial.begin(115200);
  WiFi.begin(WIFI_SSID, WIFI_PASS);
  while (WiFi.status() != WL_CONNECTED) delay(500);
  setupOTA();
  // ... rest of your setup ...
}
void loop() {
  ArduinoOTA.handle();  // CRITICAL: must be first line in loop
  // ... rest of your code ...
}

💡  Pro Tip:  To upload via OTA in Arduino IDE: after the first OTA-enabled upload via USB, the board's hostname will appear as a Port option (e.g. "my-esp32-sensor at 192.168.1.100"). Select this port instead of the COM port and click Upload. The IDE will prompt for the OTA password.

## 8.2  Deep Sleep for Battery-Powered Projects

The ESP32 normally draws 160–240 mA when running with WiFi active. A typical 18650 battery holds about 3000 mAh. Running continuously, it would last only 12–18 hours. With deep sleep, drawing just 10 µA while sleeping, the same battery can last months.

// ── ESP32 deep sleep pattern — most battery IoT devices use this ──
#include <WiFi.h>
#include <esp_sleep.h>
#include <DHT.h>
#include "secrets.h"
// RTC memory — survives deep sleep
RTC_DATA_ATTR int bootCount = 0;
RTC_DATA_ATTR float lastTemp = 0;
DHT dht(4, DHT22);
void setup() {
  Serial.begin(115200);
  bootCount++;
  Serial.printf("Boot #%d\n", bootCount);
  // Read sensor
  dht.begin();
  delay(2000);  // DHT22 needs time to stabilise
  float t = dht.readTemperature();
  float h = dht.readHumidity();
  if (!isnan(t)) {
    lastTemp = t;
    // Connect WiFi, publish MQTT, then disconnect
    WiFi.begin(WIFI_SSID, WIFI_PASS);
    int attempts = 0;
    while (WiFi.status() != WL_CONNECTED && attempts < 20) {
      delay(500); attempts++;
    }
    if (WiFi.status() == WL_CONNECTED) {
      publishMQTT(t, h);   // Your publish function
      WiFi.disconnect(true);
      WiFi.mode(WIFI_OFF);
    }
  }
  // Go to deep sleep for 5 minutes
  Serial.println("Sleeping for 5 minutes...");
  Serial.flush();  // Ensure serial data is sent before sleep
  esp_sleep_enable_timer_wakeup(5 * 60 * 1000000ULL);  // 5 min in µs
  esp_deep_sleep_start();
  // Code never reaches here — esp_deep_sleep_start() does not return
}
void loop() {
  // loop() is never called — deep sleep restarts at setup()
}

🔑  Key Concept:  RTC_DATA_ATTR declares a variable that is stored in the RTC (Real-Time Clock) memory domain, which is powered even during deep sleep. This is how you persist state (boot counter, last reading, wifi connect count) across sleep cycles.

## 8.3  Raspberry Pi — Linux Basics

### First-Time Setup with Raspberry Pi Imager

- Download Raspberry Pi Imager from raspberrypi.com/software

- Insert your microSD card (minimum 8 GB Class 10, or any U1/U3 rated card)

- In Imager: click the gear icon ⚙ to open Advanced Settings BEFORE writing

- In Advanced Settings: set hostname (e.g. "mypi"), enable SSH, set username and password, configure WiFi SSID and password

- Choose your OS (Raspberry Pi OS Lite for headless, full for desktop), click Write

- After writing, insert SD into Pi, power with USB-C (5V 3A minimum) and wait 2–3 minutes for first-boot setup

### Essential Linux Commands for RPi

# Navigation
pwd                        # Print current directory
ls                         # List files
ls -la                     # List with details (permissions, size, date)
cd /home/pi/myproject      # Change directory
cd ..                      # Go up one level
# Files and folders
mkdir projects             # Create a directory
nano myfile.py             # Edit a text file (Ctrl+X to exit, Y to save)
cp file.py backup.py       # Copy a file
mv old.py new.py           # Move or rename a file
rm file.py                 # Delete a file (CAREFUL — no recycle bin)
cat file.py                # Print file contents to terminal
# Running Python
python3 sensor.py          # Run a Python script
sudo python3 gpio.py       # Run with admin privileges (needed for GPIO)
python3 server.py &        # Run in background (& = background)
# System
sudo apt update            # Update package list
sudo apt upgrade           # Install available upgrades
sudo apt install python3-flask  # Install a package
sudo reboot                # Restart the Pi
sudo shutdown -h now       # Safely power off
df -h                      # Check disk space
free -h                    # Check RAM usage
top                        # Live process monitor (q to exit)
# Raspberry Pi specific
sudo raspi-config          # Configuration tool: enable I2C, SPI, SSH, camera
vcgencmd measure_temp      # CPU temperature
gpio readall               # Show all GPIO pin states (needs WiringPi)

### SSH — Remote Terminal Access

SSH (Secure Shell) lets you control the Raspberry Pi from your laptop over WiFi, without a keyboard or monitor connected to the Pi.

# From Windows (PowerShell or cmd), Mac, or Linux terminal:
ssh pi@raspberrypi.local     # Use mDNS hostname (works if hostname was set in Imager)
ssh pi@192.168.1.50          # Or use the IP address directly
# First connection: you will be asked to verify the host fingerprint
# Type "yes" and press Enter to continue
# To find the Pi's IP address on your network:
# Option 1: Check your router's DHCP client list
# Option 2: Use nmap: nmap -sn 192.168.1.0/24
# Option 3: Connect monitor to Pi and run: hostname -I
# Transfer files to the Pi:
scp myfile.py pi@192.168.1.50:/home/pi/
# Transfer entire project folder:
scp -r myproject/ pi@192.168.1.50:/home/pi/

## 8.4  Python GPIO on Raspberry Pi

# ── RPi.GPIO — the classic library ──
import RPi.GPIO as GPIO
import time
GPIO.setmode(GPIO.BCM)      # Use BCM (GPIO number) scheme
GPIO.setwarnings(False)     # Suppress "channel already in use" warnings
GPIO.setup(18, GPIO.OUT)    # GPIO18 as output
GPIO.setup(24, GPIO.IN, pull_up_down=GPIO.PUD_UP)  # Input with pull-up
try:
  while True:
    if GPIO.input(24) == GPIO.LOW:   # Button pressed
      GPIO.output(18, GPIO.HIGH)
    else:
      GPIO.output(18, GPIO.LOW)
    time.sleep(0.01)
except KeyboardInterrupt:
  GPIO.cleanup()   # ALWAYS reset GPIO pins on exit
# ── gpiozero — simpler, more Pythonic ──
from gpiozero import LED, Button, Buzzer, DistanceSensor
from signal import pause
led = LED(18)              # GPIO18
btn = Button(24)           # GPIO24, auto pull-up
# Event-driven programming:
btn.when_pressed  = led.on
btn.when_released = led.off
pause()  # Keep script running (waits for events)
# ── Reading from MCP3008 ADC (SPI) on RPi ──
from gpiozero import MCP3008
potentiometer = MCP3008(channel=0)   # Channel 0–7
print(potentiometer.value)           # Returns 0.0 to 1.0

## 8.5  Flask Web Server on Raspberry Pi

Flask is a lightweight Python web framework. Running a Flask server on the Raspberry Pi lets you control hardware from any browser on the same WiFi network.

from flask import Flask, render_template_string
from gpiozero import LED, CPUTemperature
app = Flask(__name__)
led = LED(18)
cpu = CPUTemperature()
HTML = """
<!DOCTYPE html><html>
<head><title>Pi Control</title>
<style>
  body { font-family: Arial; text-align: center; padding: 40px; }
  button { padding: 15px 30px; font-size: 18px; margin: 10px; }
  .on  { background: #22c55e; color: white; }
  .off { background: #ef4444; color: white; }
</style></head><body>
<h1>Raspberry Pi Control Panel</h1>
<p>CPU Temperature: {{ temp }}°C</p>
<a href="/on"><button class="on">LED ON</button></a>
<a href="/off"><button class="off">LED OFF</button></a>
</body></html>
"""
@app.route("/")
def index():
  return render_template_string(HTML, temp=round(cpu.temperature, 1))
@app.route("/on")
def turn_on():
  led.on()
  return render_template_string(HTML, temp=round(cpu.temperature, 1))
@app.route("/off")
def turn_off():
  led.off()
  return render_template_string(HTML, temp=round(cpu.temperature, 1))
if __name__ == "__main__":
  app.run(host="0.0.0.0", port=80, debug=False)
  # 0.0.0.0 means "listen on all network interfaces"
  # Access from other devices: http://192.168.1.XX  (Pi's IP)

## 8.6  Project Ideas to Build After This Workshop

| Project Name | Components Needed / Skills Practised |
| --- | --- |
| WiFi Weather Station | ESP32 + DHT22 + OLED + MQTT + deep sleep / Sessions 2, 5, 6, 7, 8 |
| Smart Plant Monitor | ESP32 + soil moisture sensor + water pump + relay + OLED / Sessions 2, 5, 6, 7 |
| RFID Door Lock | Arduino + MFRC522 + servo + OLED + buzzer / Sessions 2, 6, 7 |
| RC WiFi Car | ESP32 + L298N + servo + mobile web interface / Sessions 2, 5, 6, 7, 8 |
| LED Matrix Clock | Arduino/ESP + DS3231 RTC + MAX7219 LED matrix / Sessions 2, 6, 7 |
| Air Quality Monitor | ESP32 + MQ-135 + BME280 + OLED + ThingSpeak / Sessions 2, 5, 6, 7 |
| Home Automation Hub | RPi + MQTT broker + multiple ESP32 nodes + Node-RED / Sessions 6, 8 |
| Security Camera Alert | RPi + Pi Camera + Python + Telegram bot API / Session 8, Python |

Session Summary - OTA: always include ArduinoOTA.handle() as the first line of loop() in deployed ESP devices. - Deep sleep pattern: wake → read sensor → connect WiFi → publish → disconnect → sleep. - RTC_DATA_ATTR variables survive deep sleep. Use them for boot counters and last known values. - RPi setup: always configure hostname, SSH, WiFi in Imager Advanced Settings before writing SD card. - gpiozero is simpler than RPi.GPIO for beginners. Always call GPIO.cleanup() on exit with RPi.GPIO. - Flask on RPi: use host="0.0.0.0" to allow access from all devices on the same network.

# Appendix A — Quick Reference Cheat Sheet

## Arduino Core Functions

| Function | Description / Example |
| --- | --- |
| pinMode(pin, mode) | Set pin direction: OUTPUT, INPUT, INPUT_PULLUP pinMode(13, OUTPUT); |
| digitalWrite(pin, val) | Set digital output HIGH or LOW digitalWrite(13, HIGH); |
| digitalRead(pin) | Read digital input: returns HIGH or LOW int v = digitalRead(2); |
| analogRead(pin) | Read ADC: 0–1023 (Arduino) or 0–4095 (ESP32) int r = analogRead(A0); |
| analogWrite(pin, val) | PWM output 0–255 on ~ pins (Arduino) analogWrite(9, 128); |
| delay(ms) | Blocking wait in milliseconds delay(1000); |
| millis() | Non-blocking: ms since boot (unsigned long) unsigned long t = millis(); |
| micros() | Microseconds since boot (overflows in ~70 min) unsigned long us = micros(); |
| map(val, inLo, inHi, outLo, outHi) | Scale a value from one range to another int angle = map(raw, 0, 1023, 0, 180); |
| constrain(val, min, max) | Clamp value within range int v = constrain(val, 0, 100); |
| random(min, max) | Random number between min and max-1 int r = random(0, 100); |
| Serial.begin(baud) | Start serial comms Serial.begin(115200); |
| Serial.print / println | Send text to Serial Monitor Serial.println("Hello"); |

## Pin Quick Reference

| Feature | Arduino Uno | ESP32 DevKit |
| --- | --- | --- |
| Digital out | D0–D13 | Any GPIO except 34–39 |
| Digital in | D0–D13 | All GPIO including 34–39 |
| PWM out | D3,5,6,9,10,11 only | All output-capable GPIOs |
| Analogue in (ADC) | A0–A5 (10-bit, 5V) | GPIO32–39 ADC1 (12-bit, 3.3V) |
| True DAC out | None | GPIO25, GPIO26 (8-bit) |
| I2C SDA | A4 | GPIO21 (default) |
| I2C SCL | A5 | GPIO22 (default) |
| SPI MOSI | D11 | GPIO23 (default) |
| SPI MISO | D12 | GPIO19 (default) |
| SPI SCK | D13 | GPIO18 (default) |
| UART TX | D1 (=USB TX) | GPIO1 (UART0), configurable |
| UART RX | D0 (=USB RX) | GPIO3 (UART0), configurable |
| Logic voltage | 5 V | 3.3 V (NOT 5V tolerant!) |
| Current per pin | 40 mA max | 40 mA max (keep under 20 mA) |

## I2C Common Addresses

| Address | Device |
| --- | --- |
| 0x3C or 0x3D | SSD1306 OLED display |
| 0x27 or 0x3F | PCF8574 I2C LCD backpack |
| 0x76 or 0x77 | BME280 / BMP280 pressure sensor |
| 0x68 or 0x69 | DS3231 RTC or MPU6050 accelerometer |
| 0x40 | INA219 current sensor or PCA9685 PWM driver |
| 0x48–0x4B | ADS1115 16-bit ADC |
| 0x1E | HMC5883L compass |
| 0x29 | VL53L0X laser distance sensor |

## Baud Rate Reference

| Baud Rate | Use Case |
| --- | --- |
| 9600 | Default, most compatible, works with SoftwareSerial |
| 19200 | Some GPS and sensor modules |
| 57600 | Common for HC-05 Bluetooth AT command mode |
| 115200 | Recommended for Arduino IDE debugging and ESP boards |
| 921600 | ESP32 OTA upload, esptool flashing |
| 74880 | ESP8266 boot messages (unusual rate) |

## Resistor Values for Common Situations

| Situation | Recommended Value / Calculation |
| --- | --- |
| LED current limiting (5V, standard LED) | 220 Ω to 470 Ω / R = (5V − 2V) ÷ 0.015A = 200 Ω → use 220 Ω |
| LED current limiting (3.3V, standard LED) | 68 Ω to 150 Ω / R = (3.3V − 2V) ÷ 0.015A = 87 Ω → use 100 Ω |
| Pull-up or pull-down for GPIO | 10 kΩ / Strong enough to hold state, weak enough to not waste much current |
| I2C pull-up (SDA and SCL) | 4.7 kΩ each / Required for reliable I2C at 100–400 kHz |
| 5V to 3.3V voltage divider | R1 = 10 kΩ, R2 = 22 kΩ / Output = 5 × (22/32) = 3.44 V |
| Transistor base resistor (NPN, LED-level signal) | 1 kΩ to 10 kΩ / Depends on transistor h_FE and load current |

# Appendix B — Glossary of Terms

ADC (Analogue-to-Digital Converter)

A circuit that converts a continuous analogue voltage (e.g. from a sensor) into a discrete digital number. Arduino Uno has a 10-bit ADC (0–1023 steps). ESP32 has a 12-bit ADC (0–4095 steps).

Baud rate

The number of signal changes per second in a serial communication link. For UART, this equals bits per second. Both communicating devices must use the same baud rate.

Bootloader

A small program pre-installed in the MCU that runs before your sketch. It allows new firmware to be uploaded over USB without any external programmer. On Arduino, the bootloader occupies the last 512 bytes to 2 KB of Flash.

Brown-out detection (BOD)

A safety circuit that resets the MCU if the supply voltage drops below a safe threshold. Prevents data corruption when a battery starts dying.

Capacitor

A component that stores electrical charge. Used for decoupling (filtering power supply noise), timing circuits, RC debounce filters, and energy storage.

DAC (Digital-to-Analogue Converter)

Converts a digital number to a proportional analogue voltage. ESP32 has two 8-bit DAC outputs on GPIO25 and GPIO26.

Debouncing

The process of filtering out the rapid multiple transitions caused by mechanical switch bouncing, ensuring only one clean transition is registered per press.

Deep sleep

A very low power mode where the MCU core, WiFi, and most peripherals are powered off. The ESP32 can draw as little as 10 µA in deep sleep. The device wakes up via a timer, GPIO signal, or ULP processor.

EEPROM

Electrically Erasable Programmable Read-Only Memory. Non-volatile storage that persists across power cycles. Used for storing settings, calibration data, and configuration. Has limited write cycles (~100,000 per address).

Firmware

The compiled binary program that runs directly on a microcontroller. The term distinguishes embedded program code from application software running on a full operating system.

Flash memory

Non-volatile storage that holds your compiled firmware. Contents survive power-off. Also called program memory. Can only be erased and rewritten in blocks, unlike SRAM which can be modified byte by byte.

Floating pin

A digital input pin that is not connected to a defined voltage. A floating pin picks up noise and reads randomly as HIGH or LOW. Always use a pull-up or pull-down resistor to prevent this.

GPIO (General Purpose Input/Output)

Configurable pins on a microcontroller that can be programmed as digital input or output, and often serve secondary functions like UART, SPI, I2C, ADC, PWM, or DAC.

H-Bridge

A circuit of four transistors that allows a motor to be driven in both directions. The name comes from the H-shape of the circuit diagram. The L298N contains two H-bridge circuits.

I2C (Inter-Integrated Circuit)

A two-wire (SDA + SCL) serial communication bus that supports multiple devices on the same two wires. Each device has a unique 7-bit address. Master-controlled. Requires pull-up resistors on both lines.

IDE (Integrated Development Environment)

Software that combines a code editor, compiler, uploader, and debugger. Arduino IDE and PlatformIO are the main IDEs used in this workshop.

Interrupt

A hardware signal that causes the processor to immediately pause its current task and execute a specific function (ISR). Enables real-time response to events without constant polling.

IoT (Internet of Things)

The network of physical devices embedded with sensors, actuators, and connectivity that enables them to exchange data with other devices and systems over the internet.

ISR (Interrupt Service Routine)

The function that executes when an interrupt fires. Must be kept extremely short, must not call delay() or Serial, and variables shared with normal code must be declared volatile.

I²S (Inter-IC Sound)

A serial bus protocol for connecting audio devices like ADCs, DACs, and codecs. Used for high-quality audio streaming to speakers or from microphones.

Logic level

The voltage that represents a digital HIGH or LOW. 5 V logic (Arduino Uno) uses 5 V for HIGH. 3.3 V logic (ESP32, RPi) uses 3.3 V for HIGH. These systems require level shifting when connected together.

MCU (Microcontroller Unit)

A single chip containing a CPU, RAM, Flash, and peripherals. Used in embedded systems where cost, size, and power efficiency are priorities.

MQTT

Message Queuing Telemetry Transport. A lightweight publish-subscribe messaging protocol designed for constrained IoT devices and low-bandwidth networks. Requires a broker server.

Multiplexer (MUX)

A circuit that selects one of several input signals and forwards it to a single output. An I2C multiplexer (like TCA9548A) lets you use multiple devices with the same I2C address.

Non-volatile memory

Memory that retains its contents when power is removed. Flash and EEPROM are non-volatile. SRAM is volatile (cleared on power-off).

OTA (Over-The-Air)

A method of uploading firmware to an embedded device over a wireless network (WiFi), without physical access or a USB cable.

Pull-up / Pull-down resistor

A resistor connecting a GPIO pin to VCC (pull-up) or GND (pull-down) to define a default state when nothing else is driving the pin. Prevents floating inputs.

PWM (Pulse Width Modulation)

A technique that simulates an analogue output by rapidly switching a digital output HIGH and LOW. The ratio of ON time to period (duty cycle) determines the effective power delivered to the load.

REPL (Read-Eval-Print Loop)

An interactive programming environment where you type code line by line and see immediate results. MicroPython provides a REPL over serial for instant hardware testing.

RTOS (Real-Time Operating System)

A minimal operating system designed for embedded systems that guarantees tasks execute within predictable time bounds. FreeRTOS is available for ESP32 via the Arduino/ESP-IDF framework.

SDK / IDF

Software Development Kit / IoT Development Framework. Espressif's ESP-IDF is the professional C-based framework for ESP32/ESP8266. More powerful than Arduino framework but harder to learn.

SPI (Serial Peripheral Interface)

A four-wire (MOSI, MISO, SCK, SS) synchronous serial protocol. Full-duplex. Faster than I2C. One SS pin required per device. Used for SD cards, TFT displays, and fast ADCs.

SRAM (Static RAM)

Volatile working memory that holds running program data — variables, stack, heap. Very fast to access. Arduino Uno has only 2 KB. Cleared completely when power is removed.

UART (Universal Asynchronous Receiver-Transmitter)

A serial communication protocol using TX (transmit) and RX (receive) lines. Asynchronous — no shared clock. Both sides must be configured to the same baud rate.

Volatile (keyword)

A C/C++ keyword that tells the compiler a variable can change at any time (e.g. modified by an ISR). Prevents the compiler from optimising away reads of the variable. Required for variables shared between normal code and ISRs.

Watchdog Timer (WDT)

A hardware timer that automatically resets the MCU if the software does not periodically "kick" or "feed" it. Protects against software hangs and infinite loops.

— End of Workshop Student Notes —

Keep building. Every expert was once a beginner.